# A Systematic Literature Review about the impact of Artificial Intelligence on Autonomous Vehicle Safety

A. M. Nascimento1, L. F. Vismari1, C. B. S. T. Molina1, P.S. Cugnasca1, J.B. Camargo Jr.1, J.R. de Almeida Jr.1, R. Inam2, E. Fersman2, M. V. Marquezini3, and A. Y. Hata3

Abstract—Autonomous Vehicles (AV) are expected to bring considerable beneﬁts to society, such as trafﬁc optimization and accidents reduction. They rely heavily on advances in many Artiﬁcial Intelligence (AI) approaches and techniques. However, while some researchers in this ﬁeld believe AI is the core element to enhance safety, others believe AI imposes new challenges to assure the safety of these new AI-based systems and applications. In this non-convergent context, this paper presents a systematic literature review to paint a clear picture of the state of the art of the literature in AI on AV safety. Based on an initial sample of 4870 retrieved papers, 59 studies were selected as the result of the selection criteria detailed in the paper. The shortlisted studies were then mapped into six categories to answer the proposed research questions. An AV system model was proposed and applied to orient the discussions about the SLR findings. As a main result, we have reinforced our preliminary observation about the necessity of considering a serious safety agenda for the future studies on AI- based AV systems.

Keywords: Autonomous vehicles, safety, artificial intelligence, machine intelligence, machine learning, SLR.

- INTRODUCTION

Advances in Artiﬁcial Intelligence (AI) are one of the key enablers of the Autonomous Vehicles (AVs) development. In fact, AVs rely on AI to interpret the environment, understand its conditions, and make driving-related decisions. Thus, it basically replicates the human driver actions when driving a vehicle. In this context, AI applied to AV has become an important research topic.

AV is a safety-critical system. When operating in an undesirable way, AV can jeopardize human lives or the environment in which it operates. It has the potential to threaten the lives of its own passengers, pedestrians and people in other vehicles, and damage other transportation system elements (e.g. other vehicles and transportation infrastructure). Therefore, it is mandatory to assure AV is safe, mainly when operating on public roads in which resources will be shared with other systems (and people).

Although safety is a mandatory characteristic to AV, and although the researchers seem to agree on the importance of AI

This work was supported in part by the Research, Development and innovation Center, Ericsson Telecommunication S.A., Brazil.

1Safety Analysis Group from Polytechnic School of University of São Paulo, São Paulo, Brazil (e-mail: alexandremoreiranascimento@gmail.com).

applied to autonomous vehicles, they seem to disagree on the AIs impact on AV safety. Many researchers, in special those related to the AI community and AV manufacturers, advocate AI as one of the core elements to enhance AV safety. Their hypothesis is the automation of the driving tasks will lead to a significant reduction of the car accidents. However, other researchers, mainly in the system safety community, argue that AI can potentially jeopardize AVs safety.

This study is the ﬁrst, as far as we are aware, to map and to organize the related literature and to provide a complete view of the aspects related to both visions, and to subsidize future studies. A preliminary study on the concerns about the differences between AI and system safety mindsets impacting AV safety was published in [1]. In this non-convergent context, this paper presents a systematic literature review (SLR) aiming to present a clear picture of the state of the art of the literature in AI on AVs safety.

This paper is structured into 5 sections. Section II presents details about the research methodology used. Section III presents the data analysis results from the SLR based on the proposed methodology. Section IV proposes an AV system model that is used to orient the discussions about SLR findings. Finally, Section V presents the final remarks.

- RESEARCH METHODOLOGY

This study was performed using the systematic literature review (SLR) method. The reasons supporting the SLR use are:

(1) its established tradition as a tool to understand state-of-the- art research in technology-related ﬁelds [2]; (2) it helps to understand existing studies and supports readers in identifying new directions in the research ﬁeld [3]; and (3) it helps to create a foundation for advancing knowledge [4].

The protocol used (Figure 1) was based on the tasks suggested by [5][6] for deﬁning the research questions, identiﬁcation of search string, source selection, study selection criteria, and data mapping. Also, the protocol followed the recommendations of [7], [8], [4] and [9] for extracting, analyzing, interpreting and reporting the literature-based ﬁndings.

2Ericsson Research, Ericsson AB, Stockholm, Sweden (e-mail: rafia.inam@ericsson.com).

3Ericsson Telecommunication S.A., Indaiatuba, SP, Brazil (e-mail: maria.marquezini@ericsson.com).

(“autonomous vehicle” OR “autonomous car” OR “automated vehicle” OR “automated car” OR “self-driven vehicle” OR “self-driving” OR “driverless”)). Note that the synonyms for each one of the topics are already presented in the Boolean string previously displayed.

Different instances of the search string were created to adapt it to the distinct database search syntax rules, but the same logical value was kept. In each database, the appropriate options were selected to limit the search process to the Title-Abstract- Keyword (TAK) ﬁeld set. This is an important measure to reduce the number of non-related or duplicated studies retrieved. However, it was observed that not all databases support a search limited on TAK ﬁeld set, leading to an inﬂated number of papers found (e.g. SpringerLink). Table I shows the initial number of papers found per database.

Figure 1. Protocol used to support systematic literature review.

- Definition of Research Questions

The ﬁrst step was to deﬁne the research questions (RQ). In order to support the research goal of presenting a clear picture of the state of the art in the literature about AI on AV safety, the following research questions were posed:

  - RQ1. How do AI-based systems impact system safety?

  - RQ2. Which are the topics (context domain) of the studies

identiﬁed?

  - RQ3. Which AI-related techniques are used on the studies?

  - RQ4. Which problems do the techniques seek to address?

  - RQ5. Which ﬁndings are reported by the study’s authors?

  - RQ6. Which future studies are suggested in these studies?

- Identification of Search String and Source Selection

The search strategy was structured through the selection of source databases and the appropriate search terms. No date range was used, to ensure that relevant studies were covered, regardless of their publication date. A broad selection of online databases indexing scientiﬁc literature was considered: ACM, Engineering Village, ScienceDirect, Scopus, SpringerLink, Wiley and Web of Science (WoS). Please note that IEEExplore is already covered by the selected databases for this SLR study. The search string was designed based on the synonyms of the

3 main concepts related to the investigated topics: Safety, Artiﬁcial Intelligence and Autonomous Vehicle. Many synonyms are present in the literature for the terms "artiﬁcial intelligence" and "autonomous vehicle". Therefore, an exploratory study of their most representative synonyms was performed. Then, a careful selection of synonyms was made to ensure the search process would have an appropriate coverage. As a result, the following string with Boolean operators was selected: (“safety” AND (“artiﬁcial intelligence” OR “machine   intelligence”   OR   “machine   learning”)   AND

TABLE I

NUMBER OF PAPERS PER DATABASE

# Database	#Entries

| ACM | 36 |
| --- | --- |
| Engineering Village | 191 |
| ScienceDirect | 81 |
| Scopus | 182 |
| SpringerLink | 3999 |
| Wiley | 329 |
| WoS | 52 |
| Total | 4870 |

- Study Selection Criteria and Papers Review

The study selection process is shown in Figure 2. Each step indicates the number of papers remaining as a sample after the corresponding step was executed. The ﬁrst selection criterion applied was to ensure that only the studies with the TAK ﬁelds returning positive to the Boolean search expression would be selected. The information (metadata) available for each paper found, in the first step of the selection process, was collected by exporting the results to a spreadsheet. A spreadsheet macro was developed to analyze the TAK ﬁelds and to properly select the papers. After this check, only 230 papers remained as a sample. Using the spreadsheet Remove Duplicate tools, the duplicated entries were removed. The 97 remaining papers composed the selected sample.

Papers0 Retrieved0

(4870)0

TAK0	Duplicated0		Level050	Complete0 Filtering0		Removal0	Exclusion0		Review0

(230)0	(97)0	(86)0	(59)0

Figure 2. Study selection process.

As a reasonable number of papers (97) was found [10], book chapters, editorials, notes or reports were excluded - level 5 exclusion [10] - and 86 papers remained. The abstracts, titles and keywords of the remaining 86 peer reviewed papers were scrutinized to check their ﬁtness with the goals of this research. After a careful examination (sometimes a full-paper skimming was necessary), 27 papers were considered not related to this

research and were excluded from the sample of the literature mapping. Finally, a sample of 59 papers was considered for this study.

There was a considerable drop in the number of studies, from the initial 4870 to the final 59 papers selected. It occurred for different reasons, such as: misuse of the terminology; correct use of the terminology in the context of an example within a paper that did not actually focus on the topic; or lack of restricted search in TAK ﬁelds in some databases (in our study, the SpringerLink).

- Data Mapping

The data mapping from the selected papers were executed after they had been completely reviewed and scrutinized. It was performed categorizing the 59 sample papers into 6 categories (CT.1-6) to answer the 6 research questions (RQ.1-6), respectively. The categories deﬁned were based on the corresponding research question: (CT.1) Impact, (CT.2) Topics, (CT.3) Techniques, (CT.4) Problem, (CT.5) Findings and (CT.6) Future Studies. The categorization process was based on the agreement of researchers working in this study. Different strategies were used to create the codes for each of the categories. For (CT.1) Impact, the code increase was used when the paper described AI as a factor of increasing the safety risk (negative impact on safety) and the code decrease was used when the paper presented AI as a factor of decreasing the safety risk (positive impact on safety). For (CT.2) Topics, (CT.5) Findings and (CT.6) Future Studies, the codiﬁcations were derived by the context domain of the study according to what was reported by their authors, as suggested by [11]. Lastly, for (CT.3) Techniques and (CT.4) Problem, similarly to other categories, the codes were based on what was reported by their authors [11] and, due to the wide range of techniques, subﬁelds and misuses of terms, the terminologies were adapted and normalized according to ﬁeld references [12][13] [14][15][16][17].

- DATA ANALYSIS

The distribution of the studies over the years can provide an overview of the size and evolution of the ﬁeld (Figure 3). The left chart in Figure 3 shows the distribution from 1987 until 2018 (April). The oldest study found dates back to 1987. No work was found for over a decade – from 1991 to 2002 – considering the adopted search criteria.

This period can be labeled as the "ﬁrst winter" in this research topic as an analogy to the Artiﬁcial Intelligence "winters"1. Only one paper a year was found over the following 3 years – from 2003 to 2005. A second short winter was found from 2006 to 2008. Only 1 paper was found in 2009 and another in 2011, while no paper was found in 2010. Finally, the combination of AI, safety and autonomous vehicles started to get more attention from the scientiﬁc community in 2012 when 5 papers were found, although no paper was found in 2013. In fact, 86%

(51) of the papers found were published from 2012 to 2018.

The right chart in Figure 3 shows the distribution of the studies over the last decade. The year 2018 was excluded from the plot to avoid misinterpretation. Considering the results presented in Figure 3, the ﬁeld is gaining momentum based on the continuous growth in the number of published studies since 2014. The trend line built in the last decade data shows a higher angular coefﬁcient, indicating the momentum in recent years.

Most of the papers found are from conference proceedings. In fact, 45 papers (76%) are from conferences. Only 14 papers (24%) were published by journals. Therefore, it is reasonable to expect a growth in the number of publications about this topic in journals. Besides evaluating the time distribution of papers, another important aspect is the consistency-check of the selected keywords in the papers considered. This was performed by checking the most representative keywords among all the synonyms of each of the 3 sets (previously presented) in the search string. All the keywords from the search string found on each paper TAK were accounted. As a result, the total number of hits per keyword was computed. Table II shows the number of studies with each keyword present (hits per keyword) and the percentage of the 59 sample papers with the keyword. Note the sum of the number of hits does not totalize 59. Also, the sum of the percentages for all the keywords for each distinct concept does not totalize 100%. This is because many papers have more than 1 synonym present, which makes it be accounted more than once.

Thus, it is possible to note the most representative keyword for each concept: safety, artiﬁcial intelligence and autonomous vehicle. In fact, a search string using only those keywords would result in 36 papers, which corresponds to 61% of the sample size of the present study. However, many other keywords used could not be ignored, since they have a considerable representativeness, such as: machine learning, automated vehicle, self-driving and autonomous car. Conversely the keyword autonomous truck surprisingly had only one hit.

The following sub-sections present the results for each research question (RQ.1-6).

- AI-based systems impact on safety (RQ.1)

The RQ.1 was answered with the categorization of the sample studies into CT.1 (Impact). Most studies consider AI a technology that increases the system safety (positive impacts on safety). So, 81% (48) of the papers were actually coded as decrease, because they argue that AI decreases the safety risks. Only 19% (11) of the studies consider AI a potential threat to the system safety.

- Main topics of the studies (RQ.2)

In order to answer RQ.2, the sample papers were classiﬁed into the category CT.2 (Topics). Studies were grouped based on their CT.1 coding into two distinct sets: Increase Safety

1 The Artiﬁcial Intelligence ﬁeld had periods of warm enthusiasms and some periods of very low enthusiasm, with a much lower number of publications

and contributions. The literature named those low enthusiasm periods as AI winters.

25

20

15

10

5

0

1987	1992	1997	2002	2007	2012	2017

30

25

20

15

10

R² = 0,60323

5

0

2008      2009      2010      2011      2012      2013      2014      2015      2016      2017      2018

-5

Figure 3. Studies distribution over the years: a) depicting all studies till 2018; b) depicting studies in the last ten years.

TABLE II KEYWORDS HITS

# Concept	Keyword	#Hits      %Papers

| Safety Safety | 59 | 100 |
| --- | --- | --- |
| Artificial Artificial intelligence | 36 | 61 |
| Intelligence Machine learning | 27 | 46 |
| Machine intelligence | 1 | 2 |
| Autonomous vehicle | 37 | 63 |
| Autonomous Automated vehicle | 12 | 20 |
| Vehicle Self-driving | 11 | 19 |
| Autonomous car | 6 | 10 |
| Driverless | 2 | 3 |
| Autonomous truck | 1 | 2 |

Risks and Decrease Safety Risks. Then studies were grouped by their similarities and each group was coded with a label that could encompass all its members. Table III shows the results of this coding process. As observed, the papers positioning AI as a factor that decreases safety risks (48 papers, 81%), they studied the subjects related to five main topics: Sensors and Perception (21 papers, 44%), Navigation and Control (13

papers, 27%), Fault Prevention (6 papers, 13%), Conceptual Model and Framework (4 papers, 8%) and Human Factor (4 papers, 8%). In turn, the papers positioning AI as a risk to system safety (11 papers, 19%) studied subjects related to three main topics: Fault Forecasting (5 papers, 45%), Ethics and Policies (4 papers, 36%) and Dependability and Trust (2 papers, 18%). The complete list of references for each code in this category can also be found in Table III.

The main topics for each group of papers differ reasonably from each other. While the papers in the category decrease focus on important aspects to support or to enhance the vehicle autonomy, the papers in the category increase (endanger safety) focus on topics related to safety assurance.

Sensors and Perception is the topic with the largest number of studies (21). They are mostly related to computer vision and detection techniques necessary for adding the necessary capabilities to detect different aspects of the navigation environment and supporting the autonomy of the AVs, such as: general computer vision [19], Doppler sensing [20], lane detection [21], daylight detection and evaluation [22], obstacles detection [23][24][25], pedestrian detection [26][27], pedestrian trajectory prediction [28], road detection [24][29][75], road junction detection [30], road terrain detection [31], trafﬁc signal detection [32][18], situation awareness [33], speed bump detection [25][34], trafﬁc light

detection [35], vehicle detection [36] and virtual worlds for training detection [37].

The second largest number of papers (13) found encompasses studies related to Navigation and Control. They are mostly related to techniques necessary to ensure the proper autonomous navigation and control capabilities required by AVs, such as: remote-controlled semi-AV based on IoT [38]; adaptive pre-crash control [39]; safe trajectory selection [76]; AV following another car driven by a human pilot (Trailing) [40]; safe navigation [41]; heuristic optimization algorithm for unsigned intersection crossing [42]; vehicle coordination [43]; maneuver classiﬁcation [44]; learning to navigate from demonstration [45]; AV movements optimization in intersection [46]; learning and simulation of the Human Level decisions involved in driving a racing car [47]; path tracking [48]; and fuzzy-logic control approach to manage low level vehicle actuators (steering throttle and brake) [49].

Six papers with research related to Fault Prevention were found. These studies encompass researches related to the preventing the occurrence or introduction of faults [50], such as AI for security of wireless communication to ensure safety [51]; remote diagnosis, maintenance and prognosis Framework [52]; prediction of computational workload [53]; vehicle security against cyber-attack [39][54]; and diagnosis of sensor faults [55].

Four studies were found for each of the topics Human Factor and Conceptual Model and Framework. The studies on human factor cover important aspects to be considered in the autonomous cars engineering due to the human-in-the-loop factor, such as: safety, comfort, and stability based on the human driver perception behavior [56]; design of real time transition from assisted driving to automated driving under conditions of high probability of a collision [57]; diagnosing and predicting stress and fatigue of driver in semi-automated vehicles [58]; and advances in driver-vehicle interface [59]. Considering the studies (4) proposing conceptual models and frameworks, they have a considerable diversity of focus, such as: ML and cloud-based framework proposed to address safety and reliability-related issues [60]; AV conceptual model [61]; an interdisciplinary framework to extract knowledge from the large amount of available data during driving to reduce driver’s behavioral uncertainties [62]; and a proposition of an AV highway concept to improve highway driving safety [63].

Considering the group of papers positioning AI as a potential factor of decreasing the safety, the highest number of studies was related to Fault Forecasting. In other words, those

| TABLE III IMPACT OF AI-BASED SYSTEMS ON SAFETY AND ITS MAIN TOPICS AND REFERENCES |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category | Codes | #Hits | %Papers | References |
| CT.1 - Impact | Decrease Safety Risks (Positive Impact on Safety) | 48 | 81 |  |
|  |  |  |  | [18][75][35][36][21][25][27][29] |
|  | Sensors and Perception | 21 | 44 | [23][22][34][20][44][32][31][33] |
|  |  |  |  | [26][28][30][24][19] |
| CT.2 - Topics | Navigation and Control | 13 | 27 | [38][39][76][40][41][47][42][43] [48][44][45][46][49] |
|  | Fault Prevention | 6 | 13 | [39][51][52][53][54][55] |
|  | Conceptual Model and Framework | 4 | 8 | [60][61][62][63] |
|  | Human Factor | 4 | 8 | [56][57][58][59] |
| CT.1 - Impact | Increase Safety Risks (Negative Impact on Safety) | 11 | 19 |  |
|  | Fault Forecasting | 5 | 45 | [64][65][66][67][68] |
| CT.2 - Topics | Ethics and Policies | 4 | 36 | [69][70][71][72] |
|  | Dependability and Trust | 2 | 18 | [73][74] |

papers dealt with the limitations to estimate the present number and future incidence of faults in AI-based systems, by executing activities related to evaluation, testing, veriﬁcation and validation [50], such as: aspects (and limitations) related to safety validation [64]; performance and safety veriﬁcation methodology [65]; test suites for AV [66]; end-to-end safety for AV design [67]; and a framework to evaluate the impacts of such a sophisticated system on trafﬁc and the impact of continuous increase in the number of highly automated vehicles on future trafﬁc safety and trafﬁc ﬂow [68].

There were four studies related to discussions about Ethics and Policies. One of the studies discussed and performed experiments on how distinct ethical frameworks adopted to make decisions about AV crashes can affect the number of lives endangered [69]. The other studies discuss the scope of AI on AV with ethical aspects [70], ethics in AV design [71], and moral values and ethical principles for autonomous machines [72]. As can be seen, those studies are quite recent since the oldest one was published in 2015.

Finally, 2 papers were found related to Dependability and Trust. Dependability is an important concept in critical systems, because it comprises attributes such as safety, security, availability, reliability and maintainability, as well as how (the mechanisms) to keep these systems attributes [50]. According to [50], trust can be deﬁned as accepted dependability. The studies found are thus related to: safety issues [73] and current mechanisms to ensure robust operation in safety-critical situations facing the introduction of non-deterministic software [74].

- Techniques used (RQ.3) and problems they seek to address (RQ.4)

Aiming to answer RQ.3 and RQ.4, the sample papers were classiﬁed into categories CT.3 (techniques) and CT.4 (problems) based on how their authors described the AI technique used in the study. Then, some terminologies used to define the codification for the categories CT.3 and CT.4 were adapted based on the ﬁeld literature [12][13][14][15][16][17], when necessary.

Most reviewed papers reported the speciﬁc AI related techniques used in the research. Some reported the use of more than one technique, whereas others reported only the approach used. Some papers (14 papers, 24%) were related to general aspects of AI or ML techniques, without mentioning speciﬁc techniques used or researched [68][71][72][61][64] [67][69][74][70][65][63][58][62][19].

All the techniques found in the reviewed papers were mapped considering the problem (CT.4) that they were solving. As a result, Table VIII - placed in the Appendix - lists the techniques found, the number of papers in which they were used, the main problems they were seeking to address, and the references.

As can be seen, there is a considerable number of studies (22%,13) that used techniques related to artiﬁcial neural networks. Also, there is a reasonable number of studies reporting the use of SVM (17%, 10). Some studies used Fuzzy Logic (8%, 5), Bayesian Artiﬁcial Intelligence (e.g. Bayesian Deep Learning, Naive Bayes Classiﬁer-NBC, etc) (7%, 4), Hidden Markov Based Models (e.g. Continuous Hidden Markov Model-CHMM and Discrete Hidden Markov Model DHMM) (7%, 4), Estimation Filters (e.g. Kalman Filter and Particle Filters) (7%, 4), Nearest-Neighbour-Based Algorithm (e.g. k-Nearest Neighbours - kNN) (7%, 4), Adaptive Boosting (AdaBoost) (5%, 3), Ramer-Douglas-Peucker or Rameri

Douglas algorithm (5%, 3), Haar-like feature detector (5%, 3), Histogram of Oriented Gradient (HOG) (5%, 3), Hough Transformation (5%, 3), Optimization Heuristics (5%, 3), Regression-Based Models (5%, 3) and Principal Components Analysis (PCA) (3%, 2).

Analyzing Table VIII, it shows that each of the following techniques were reported, in all the reviewed papers, only once: Canny Edge Detection Algorithm, Case-based reasoning (CBR), Channel Features, Clustering Algorithm k-mean, Complex Decision Trees (CDT), Conditional Random Fields (CRFs), Distributed Random Forest (DRF), Gaussian Mixture Model (GMM), Linear Temporal Logic (LTL), Local Binary Patterns (LBP), Neuroevolution of Augmenting Topologies (NEAT), Novel Image Recognition Technique, Path Planning Algorithms (A* and D*), Satisﬁability Modulo Theories (SMT)

Solver, and Viterbi Algorithm. Thus, there is room for new studies using techniques not yet used or under-represented by the set of papers considered.

- Reported findings (RQ.5)

Question RQ.5 is answered by CT.5 (ﬁndings), based on the information about the ﬁndings reported on the sample papers. Some papers did not report speciﬁc main ﬁndings in a straightforward way because the propose frameworks or approaches had not yet been tested or the results were still incipient. Other papers described very speciﬁc ﬁndings that would require a background section to support a proper discussion. In those cases, only a higher level of abstraction of the results is presented. Finally, because of space limitation, only some speciﬁc examples are described here, while most of the results are presented grouped around the main topic of research. A complete list, oriented by the discussion presented at Section IV, can be found on the Table IX presented in Appendix.

The papers about topics related to Sensors and Perception presented positive and promising results with the techniques employed to address their research problems. In fact, this topic already achieved significant results with the recent developments in AI and sensor technologies. While AI had the image and pattern recognition boosted by advancements such as the new architectures of ANNs and new machine learning techniques, sensor technologies have been boosted in the last decades by the advancements in the robotics and mobile phone industries. As a result, the papers demonstrated applications of enhancements in the techniques or combination of techniques and sensors in order to recognize and to detect important elements and signals the human drivers need to handle to ensure the proper operation of a vehicle. In this context, the ﬁndings are positive for the application of ANNs to recognize turn signal [18], road environment and signals [27][32][31][30], and pedestrian [26][28], for example. Likewise, some papers reported SVM has been applied successfully to detect road [75], trafﬁc light [35], and pedestrian [27].

The papers related to Navigation and Control also reported positive and promising results. As presented previously, they used diverse AI techniques to seek to address a broad range of problems. For example, a hybrid AI architecture encompassing ANN, CBR, and a hybrid Case-Based Planner (A* and D* motion planner) was successfully tested to tackle the precrash problem of intelligent control of autonomous vehicles [39], while SVM was used to support a safest path planning in a dynamic environment to avoid maneuvers too close to an obstacle [41].

This SLR found 6 papers for the topic Fault Prevention. Each of these papers used a distinct AI technique for the research problems. One paper presented a preliminary result [53], and another one proposed an approach but did not report results [55]. All the others papers, related to the detection of cyber- attack, presented promising positive results for the application of ANNs [39], Estimation Filters [51], and Fuzzy-Logic [54], for example. Also, preliminary positive results have been reported on the use of a regression-based model to predict the

CPU patterns [53].

Two from the four remaining papers related to the topic Human Factor, have presented preliminary positive results. One presented promising results from using a regression-based model to deal with selective attention mechanism [56], while the other presented some examples of scenarios where the use of Bayesian AI could avoid the collision when no action is taken by the human driver [57]. The other 2 papers did not present speciﬁc ﬁndings, due to their theoretical nature related to the design considerations for the driving assistance system [59] and human drivers monitoring to enhance the integration between AVs and human drivers [58].

The papers proposing conceptual models and frameworks did not present ﬁndings related to experimental results. Most of them relied on general AI/ML instead of a speciﬁc technique [61][62][63]. Also, besides the proposed approaches themselves, they focused the discussions around the issues they aimed to address, the theoretical background and future potential problems to be addressed in the ﬁeld.

The last three topics (Fault Forecasting, Ethics and Policies, and Dependability and Trust) have papers more oriented to theoretical discussions and propositions around the challenges AVs are facing or will face related to safety topics, such as test and validation [64], certiﬁcation [67][74], autonomy assurance and trust when non-deterministic and adaptive algorithms are used [74] - crash assignment facing distinct ethical theories [69], for example. In this context, most of them do not present speciﬁc ﬁndings using experimental setups; instead, they envision potential future solutions for the discussed challenges. In other words, those papers try to shed an alert light on the important topics that seem to be neglected by the AV enthusiasts, trying to push the research agenda towards safety engineering mindset.

As exceptions, 3 papers presented practical applications and results. [69] presented some interesting ﬁndings using a simple experimental simulated environment to test speciﬁc crash scenarios under three ethical theories. They found that understanding rational ethics is crucial for developing safe automated vehicles. The results of their experiment indicate that in speciﬁc crash scenarios, utilitarian ethics may reduce the total number of fatalities that result from automated vehicle crashes. [66] proposed an approach to describe test-cases for validating autonomous vehicles using recordings of trafﬁc situations for creating a minimal test-suit that could help in the certiﬁcation process. Considering the example presented, they show how minimalism is achieved by manually comparing the test-cases. Although it is an interesting and promising approach, there are no evidences that it could address a safety certiﬁcation processes requirement when considering non-deterministic algorithms. Hence, the research was still preliminary. Finally, although [73] presents an end-to-end Bayesian Deep Learning architecture to reduce the risks of hard classiﬁcations by adopting probabilistic predictions accounting for each model, no ﬁndings from real experiments were presented.

- Reported future studies (RQ.6)

Question RQ.6 is answered by CT.6 (future studies), based

on the collected information about future studies reported on the sample papers. Some papers did not suggest future studies. Other papers described intended future studies or works under development. Those are frequently small incremental changes, such as change of parameter or new test scenarios. Therefore, they are not reported here since their speciﬁcities would require a considerable background on the papers contents. That is out of the scope of the systematic literature review.

The studies related to Sensors and Perception propose many future studies, but mostly around improvements that would be made in the future to address some of the limitations of the presented research. Due to the space limitations, only some examples are described here. [18] suggests additional research on image recognition of low contrast images and vehicle images with brake lamps. [35] suggests future work on traffic lights detection under severe weather or night conditions. [34] suggests more research on detecting speed bump during night time. They also suggest research on speed bumper detection when they have no pattern or marking. In addition to that, [34] suggests research to improve the recognition capabilities to distinguish zebra crossing from speed bump. [75] proposed future research about road detection using road lane markers that could be detected by LIDAR, while [21] proposed more research focused on optimizing the lane detection and vehicle recognition algorithms to reduce their computational costs. Also considering the high computational costs, [27] proposed using parallel computing to increase the speed of the image recognition algorithms. Finally, according to [37], additional research is needed on using the virtual environments for testing because the authors believe their usage for training and testing intelligent systems are becoming more relevant.

Most of the studies related to Navigation and Control suggest future studies. The majority suggests extensions to the work they presented. Here, few examples are presented. The study proposing hybrid control architecture [39] suggests an extension to consider the full kinematics and dynamic limitations of the vehicle, while constantly acting to avoid collisions and unsafe driving. The paper proposing an approach using SVM to avoid maneuvers too close to an obstacle by adding a safety margin [41] proposes future re-search to extend it using a combination with the kinetic convex hulls2 to enable the possibility of computing the solution ahead in time. According to the authors, this would help to predict the position and the width of the optimal margin. As a result, it would improve the approach by adding the ability of reduce the collision risk by preventing the AV from driving into a dangerous situation. The study using Fuzzy Logic as the main approach to control a semi-autonomous car 100-km experiment

[40] proposes future research using new sensors and ﬁltering methods for data fusion to reduce the risk on scenarios where the GPS signal is lost. Finally, the study on AVs intersection crossing [46] describes future work in which more types of vehicles and more adjacent intersections would be included in the simulations.

Most of the studies (4 of 6) related to Fault Prevention

2 Check [76] for more information about kinetic convex hulls.

suggest future studies. Half of the studies are related to security aspects, while the other half is related to diagnosis/prognosis/prediction. The study proposing a cyber- attack detection system based on ANNs [39] suggests a future study to apply the proposed approach to a real vehicle in addition to the application of LSTM to detect online sensor attack. The study proposing the use of Particle Filter and Kalman Filter to secure connected vehicles against DoS attack

[51] proposes future work to assess the proposed security scheme under many distinct scenarios, and also to execute tests in real world set-ups. The study about predicting ADAS remaining useful life for the prognosis of its safety critical components using ANNs and other techniques, such as SVM [52], proposes a considerably wide range of future studies, such as using Least Square Support Vector Machine (LSs SVM); using big data techniques to analyze the server data; studying connected vehicle prognosis; using driver, vehicle and region proﬁle data to understand the impact on the environment and driving style impact on the system lifespan; and more studies on prognostics-enabled decision Making (PDM). Finally, the work presenting the use of regression-based methods to predict the CPU usage patterns of software tasks running on an AV [53] suggest future work on the use of some regularization methods for automatic feature selection, but also to particularly investigate the effects of underestimating CPU utilization, and how to handle underestimation of CPU utilization when it happens, aiming to better understand how safe over (or under) estimation of CPU utilization is in terms of reliable autonomous driving.

The studies about Ethics and Policies on AVs basically suggest more research on those topics. In the same way, most of the studies tackling human-factor-related topics do not propose future studies. As an exception, the paper proposing the application of regression-based model for the selective attention mechanism subject [56] proposed a future study to help to reveal the mechanism of rear end collision accident to some extent.

Half (2 of 4) of the studies related to Conceptual Model and Framework do not suggest any future studies. However, implicitly, the next steps would be the deployment of those suggested approaches on experimental set-ups to collect real results. The study proposing a framework to reduce the uncertainty of a driver behavior prediction model [62] suggests more studies focusing on the resilience and sustainability of the system when deployed on a large scale in a complex system.

The papers about Fault Forecasting suggest some future research. Among them, [64] suggests more research on safety envelope mechanisms to describe a boundary within the state space of the AVs rather than trying to prove that it will always work correctly. Koopman, in another paper [67], suggest that the accepted practices must be updated to create an end-to-end design and validation process to address all the safety concerns considering cost, risk, and ethical considerations. [66] proposes more work on creating automated test-cases. [68] proposes more studies based on the framework they proposed to evaluate

the impacts of AVs on trafﬁc safety, specially using stochastic simulations with random number seeds to achieve a broader representative and a variety of trafﬁc situations, as well as using the proper statistical analysis techniques to ensure the statistical validity of the results.

Finally, the 2 studies about Dependability and Trust also present some suggestions of future studies. [73] asks for more research on new concrete safety evaluation metrics. [74] suggests more research on understanding the dependence of the system components on AVs is needed to establish trust. They also suggest that could be achieved by investigating the many ways in which people, the system, and the environment interrelate.

- SLR FINDINGS ORIENTED BY AN AV SYSTEM MODEL In the previous section, the state of the art in the literature

about AI on AV safety was identiﬁed and investigated by means of a SLR. Six research questions oriented the literature identiﬁcation, in which studies that include keywords related to safety, AI and AV were considered. The resulting studies were investigated and mapped into 6 categories: Impact (increase or decrease safety risks), Topics (sensors and perception; navigation and control; fault prevention; conceptual model and framework; human factor; etc.), Techniques (general AI/ML; ANN; SVM; etc.), Problem (AV validation; road detection; collision avoidance; etc.), Findings, and Future Studies.

These results considered the AV as a system, but its speciﬁc components and functions in an architectural point-of-view were not considered. For deepening the understanding about the state of the art of AI on AV safety it is necessary to show how the presented works are applied/fitted on AV in an architectural point-of-view. In other words, which of AV modules/components and functions are already being developed and which one could be more explored. In order to achieve this goal, it is going to be considered the AV architecture proposed in [78].

An automotive manufacturer consortium (CAMP-AVR) [78] proposed a high-level architecture considering the main system components demanded for the vehicle movement control, to be used in the deployment of future Dynamic Driving Tasks (DDT). Figure 4 (left) illustrates the model considering a traditional vehicle (i.e. human operation with no automation deployed), and Figure 4 (right) illustrates the introduction of some level of machine automation (hybrid) in Sensors, Controller and Actuators elements. While the diagram considering the human operation can solely be mapped to the SAE Automation Level 0 (no automation), the hybrid one

encompassing machine automation with human-in-the-loop can be mapped to the SAE Automation Levels 1 to 4 (semi- autonomous) [78].

In this context, a modiﬁed version of the semi-autonomous model is proposed here (Figure 5) including the system boundary. Also, the human related components were grouped as one single component (human-in-the-loop), which interacts with Machine Actuators, Machine Control, Machine Perception and Environment. A single component represents a more realistic approach facing the complexity added by the human in the system and allows the examination of the user actions and interactions as suggested by [79]. Also, it supports a necessary human-centered and holistic view [80] to better support the complexity of the human behavior and its interaction to the system. It avoids the misconceptions of the too logical designs from some engineering designs and helps to consider and accept human behavior the way it is, not the way engineers would wish it to be [81]. In fact, this is a necessary upgrade considering the original model is derived by the classical view from the automation engineering for industrial applications, where the environment was under control of the system designer, the human interactions had a considerable narrower scope, and its potential impact to the whole system were much lower, when compared to its application to the semi-autonomous vehicles. As a result, the proposed DDT version (Figure 5) can be used to map the selected scientiﬁc literature. Therefore, it can provide a concise perspective on how the ﬁeld literature covers those main components and which the uncovered areas are. Also, it can provide a good overview on the predominance of the papers valence (increase or decrease) on safety.

Table IV shows how CT.1 (Impact) and CT.2 (Topic) codes are mapped to the components of the modiﬁed semi- autonomous system model, as well as the relationship between CT.1 and CT.2. Most of the papers are related to machine perception, followed by papers related to a broad system view. Then, the next largest group of papers is related to the machine control component. The remaining papers are related to the human-in-the-loop aspects. An interesting aspect is that only the studies with a broad system aspect were found to have both CT.1 codes (increase and decrease system safety). Basically, the studies focused on distinct components solely understand AI can increase the safety risk. Therefore, there is a lack of studies with a critical mindset that explore the potential negative impacts of AI on the individual components. Finally, no papers were found related to the vehicle, machine actuators or environment.

Figure 4. Dynamic Driving Tasks Models: No Automation (left) x Semi-Automation (right) – Source: [78]

different heterogeneity in the applied AI approaches. Besides 24% of the papers using generic AI/ML concepts, 49% of the papers applied only one type of AI technique. Therefore, they are homogeneous in terms of the applied AI technique. In those studies, the most widely used techniques were Artiﬁcial Neural Networks (28%, 8), Fuzzy Logic (14%, 4) and Support Vector

Machine (SVM) (10%, 3). The remaining 27% employed a hybrid approach by combining multiple types of AI techniques. Among those papers, the combination of Artiﬁcial Neural Networks to other techniques (44%, 7), Support Vector Machine to other techniques (SVM) (25%, 4) and Hidden Markov-Based Models (e.g. Continuous Hidden Markov Model-CHMM and Discrete Hidden Markov Model-DHMM) to other techniques (13%, 2) were the most frequent hybrid approaches found in the papers selected.

Figure 5. An adapted version of Dynamic Driving Task (DDT) Model

Table V shows how the wide range of AI techniques (CT.3

TABLE IV

MODIFIED SEMI-AUTONOMOUS DDT SYSTEM MODEL X CT3 CODES

%

CT.3 - Topic	Component of the Modified

DDT System Model (#Hits)

CT.2 – Impact on Safety: Increase Safety (+)

code) is mapped to the components of the modiﬁed semi- autonomous system model. The AI techniques are grouped around their scope: system-oriented (32%, 19) and component-

oriented (68%, 40). When a paper uses a combination of

Sensors and Perception Navigation and Control

Machine Perception (21)	36

Machine Control (13)	22

techniques, for example, ANN and SVM, it results into a unit added to the total number of papers using ANN and a unit added

Human Factor	Human-in-the-loop (6)	10

Fault Prevention

to the total number of papers using SVM. In this context, most of the studies (63%, 12) related to system-wide scope referred

Conceptual Model

and Framework

System (+) (8)

to general AI/MI. Most of the studies (20%, 11) related to machine perception used ANNs. In fact, ANN, SVM and HMM

CT.2 – Impact on Safety: Decrease Safety (-)	 Fault Forecasting		32

(Hidden Markov Model) account for 48% of the studies related to machine perception. Fuzzy logic (18%, 3) is the most widely used technique in the machine control-related papers. Fuzzy

Ethics and Policies

Dependence and Trust

System (-) (11)

Logic, SVM, Optimization Heuristics and Ramer-Douglas- Peucker or Ramert Douglas algorithm account for 53% of the studies related to machine control. Finally, Bayesian Artiﬁcial Intelligence techniques are used in most of the studies (29%) related to human-in-the-loop.

Table VI shows the total count of each AI technique occurrence over the sample papers. The sample papers have

Many different combinations of ANNs with other techniques were found (7 papers). As shown in Table VII most of those papers are related to Sensors and Perception (3 papers) as well as Navigation and Control (2 papers). Also, papers related to Conceptual Model and Framework and Fault Prevention employed hybrid approach (2 papers). The papers that used a

combination of models associated to Hidden Markov Based Models were related to Navigation and Control as well as Sensors and Perception. The paper that used Hough Transformation combined to other models is related to Navigation and Control. The paper that employed a combination of techniques to propose a Novel Image Recognition Technique is related to Sensors and Perception. The paper using Regression- Based Models combined to other techniques is related to AV Navigation and Control. Finally, all the papers employing SVM combined to other techniques were related to the topic Sensors and Perception. The same grouping strategy applied to Table V (system-oriented and component- oriented) can be applied here to evaluate the problems (CT.4). System-level problems include 16 papers: AV Validation [64], Machine-learning-based systems validation to the ultra- dependable levels required for AV [67], Human and Machine

Driver Co-existence [60], Coexistence Human Machine Controller [70], Driving Car Tasks Classiﬁcation [61], Lack of efﬁcient Safety Performance Veriﬁcation technique when AI/ML is used [65], Crash assignment, especially between automated vehicles and non-automated vehicles [69], Reduce the uncertainty of a driver behavior prediction model [62], Investigate three underexplored themes for AV research: safety, interpretability, and compliance [73], How vehicle autonomy technology can be used to beneﬁt car drivers and also to propose a concept of an autonomous highway vehicle which improves highway driving safety [63], AV decisions in complex dilemmas as a social agent [71], Hybrid (humans and machines) collective decision-making systems [72], Autonomy assurance and trust in Automated Transportation Systems [74], AV Test

[66] and, Evaluate the impacts of the number of highly automated vehicles on future trafﬁc safety and trafﬁc ﬂow [68].

|  | TABLE V TECHNIQUES X DDT SYSTEM MODEL COMPONENT |  |  |  |
| --- | --- | --- | --- | --- |
| DDT System Model | Technique | #Hits | %Paper | Accum.% |
|  | General AI/ML | 12 | 63% | 63% |
|  | Hough Transformation related approaches | 2 | 11% | 74% |
| System-oriented | Artificial Neural Networks | 2 | 11% | 84% |
| (System) | Optimization Heuristics | 1 | 5% | 89% |
|  | Estimation Filters (e.g. Kalman Filter and Particle Filters) | 1 | 5% | 95% |
|  | Linear Temporal Logic (LTL) | 1 | 5% | 100% |
|  | Artificial Neural Networks | 11 | 20% | 20% |
|  | Support Vector Machine (SVM) | 8 | 15% | 35% |
|  | Hidden Markov Based Models (e.g. Continuous Hidden Markov Model-CHMM and Discrete Hidden Markov | 4 | 7% | 43% |
|  | Model-DHMM) |  |  |  |
|  | Estimation Filters (e.g. Kalman Filter and Particle Filters) | 3 | 6% | 48% |
|  | Histogram of Oriented Gradient (HOG) | 3 | 6% | 54% |
|  | Nearest-Neighbor Based Algorithm (e.g. k-Nearest | 3 | 6% | 59% |

| Neighbours - kNN) Adaptive Boosting (AdaBoost) | 3 | 6% | 65% |
| --- | --- | --- | --- |
| Principal Components Analysis (PCA) | 2 | 4% | 69% |
| Haar-like feature detector | 2 | 4% | 72% |
| Fuzzy Logic | 2 | 4% | 76% |
| Viterbi algorithm | 1 | 2% | 78% |
| Bayesian Artificial Intelligence (e.g. Bayesian Deep | 1 | 2% | 80% |

| Learning, Naive Bayes Classifier-NBC, etc) |  |  |  |
| --- | --- | --- | --- |
| Regression Based Models | 1 | 2% | 81% |
| Hough Transformation related approaches | 1 | 2% | 83% |
| Ramer-Douglas-Peucker or Ramer-Douglas algorithm | 1 | 2% | 85% |
| Novel Image Recognition Technique | 1 | 2% | 87% |
| Gaussian Mixture Model (GMM) | 1 | 2% | 89% |
| General AI/ML | 1 | 2% | 91% |
| Complex Decision Trees (CDT) | 1 | 2% | 93% |
| Channel Features | 1 | 2% | 94% |
| Local Binary Patterns (LBP) | 1 | 2% | 96% |
| Clustering algorithm k-mean | 1 | 2% | 98% |
| Conditional Random Fields (CRFs) | 1 | 2% | 100% |
| Fuzzy Logic | 3 | 18% | 18% |
| Machine Support Vector Machine (SVM) | 2 | 12% | 29% |
| Control Optimization Heuristics | 2 | 12% | 41% |
| Ramer-Douglas-Peucker or Ramer-Douglas algorithm | 2 | 12% | 53% |
| Case-based reasoning (CBR) | 1 | 6% | 59% |

Component- oriented

Machine Perception

Nearest-Neighbor Based Algorithm (e.g. k-Nearest Neighbours - kNN)

1	6%	65%

Basic AI Path Planning algorithms such as A* and D*	1	6%	71% Artificial Neural Networks	1	6%	76%

Regression Based Models	1	6%	82%

Distributed Random Forest (DRF)	1	6%	88%

Neuroevolution of Augmenting Topologies (NEAT) - ANN + GA

1	6%	94%

Satisfiability Modulo Theories (SMT) Solver	1	6%	100%

Bayesian Artificial Intelligence (e.g. Bayesian Deep Learning, Naive Bayes Classifier-NBC, etc)

2	29%	29%

Human-in- the-loop

Regression Based Models	1	14%	43%

Haar-like feature detector	1	14%	57%

Canny Edge Detection Algorithm	1	14%	71%

Hough Transformation related approaches	1	14%	86%

General AI/ML	1	14%	100%

Considering the component-level problems, 21 papers (36%) are related to dealing with algorithms and techniques to deal with Machine Perception issues, such as: Vehicle Cyber Attack [39], Turn Signal Recognition [18], Securing connected vehicles against Denial of Service (DoS) attack [51], Road Detection [75], Trafﬁc Light Detection [35], Prediction of advanced driver assistance systems (ADAS) remaining useful life (RUL) for the prognosis of ADAS safety critical components [52], Vehicle Detection and Counting [36], predicts the CPU usage patterns of software tasks running on a self-driving car [53], a safety warning and driver-assistance system and an automatic pilot for rural and urban trafﬁc environments [21], reliable and robust obstacles detection continues to be largely investigated and still remains an open challenge, especially for difﬁcult scenarios and, in general cases, with loosened constraints and multiple simultaneous use- cases [25], Pedestrian Detection [27], Road environmental recognition and various object detection in real driving conditions [29], Obstacle clustering and tracking [23]. For an autonomous behavior, each truck must be able to follow the vehicle ahead. Due to that, each vehicle must be able to recognize the leading vehicle [22], Speed bump detection [34], providing road safety to connected drivers and connected autonomous vehicles [20], how to ”automate” manual annotation for images to train visual perception for AVs [44], Road Sign Classiﬁcation in Real-time [32], Road Terrain detection [31], Spatio-temporal situation awareness [33], Pedestrian detection and movement direction recognition [26], Pedestrian Trajectory Prediction [28], Road junction detection [30], Cyber Attack in V2X [54], Learn from Demonstration [45], Early detection of faults or malfunction [55], Road and Obstacle Detection [24] and Enhance Image Understanding [19].

The problems related to Machine Control were found in 17 papers (22%). Those problems include: Pre-Crash problem of Intelligent Control of autonomous vehicles robot [39], Safe- optimal trajectory selection for autonomous vehicle [76], Driverless car 100 km experiment [40], Robot maneuvers too close to an obstacle, which increases the probability of an accident. Preventing this is crucial in dynamic environments, where the obstacles, such as other UAVs, are moving [41], Learning and simulation of the Human-Level decisions involved in driving a racing car [47], Control intersection crossing (all way stop) and optimizing it [42], How to prove the correctness of an algorithm for Vehicle Coordination [43], Path tracking [48], Drivers maneuver classiﬁcation [44], AVs intersections crossing optimization [46] and Manage low level vehicle actuators (steering throttle and brake) [49].

Finally, the problems related to Human-in-the-loop new DDT component (Figure 5) are present in 7 papers (10%). Those problems include: Selective Attention Mechanism [56], Developing remote controlled car with some automation to deal with trafﬁc light detection, obstacle avoidance system and lane detection system to be driven from anywhere over a secured internet connection [38], Collision avoidance when no action is taken by driver to avoid the collision [57], Human drivers monitoring system to ensure they will be able to take over control within short notice [58] and, Design of driving assistance system [59]. This seems to be an attention-point; this problem category can be considered one serious challenge to semi-autonomous vehicles (SAE Level 1 to Level4). Therefore, more research is needed into this topic because only 6 papers were found.

TABLE VI

HETEROGENEITY OF THE USED AI APPROACHES

| Heterogeneity | % | Main Technique | #Hits | %Papers |
| --- | --- | --- | --- | --- |
| Generic | 24% | General AI/ML | 14 | 100% |
|  |  | Artificial Neural Networks | 8 | 28% |
| Homogenous | 49% | Fuzzy Logic | 4 | 14% |
|  |  | Support Vector Machine (SVM) | 3 | 10% |

| Regression Based Models |  |  | 2 | 7% |
| --- | --- | --- | --- | --- |
| Estimation Filters (e.g. Kalman Filter and Particle Filters) |  |  | 2 | 7% |
| Bayesian Artificial Intelligence |  |  | 2 | 7% |
| Optimization Heuristics |  |  | 2 | 7% |
| Ramer-Douglas-Peucker or Ramer-Douglas algorithm |  |  | 2 | 7% |
| Hough Transformation |  |  | 1 | 3% |
| Satisfiability Modulo Theories (SMT) Solver |  |  | 1 | 3% |
| Adaptive Boosting (AdaBoost) |  |  | 1 | 3% |
| Linear Temporal Logic (LTL) |  |  | 1 | 3% |
| Artificial Neural Network combined to other techniques |  |  | 7 | 44% |
| Support Vector Machine (SVM) combined to other techniques |  |  | 4 | 25% |
| Hybrid | 27% | Hidden Markov Based Models (e.g. Continuous Hidden Markov Model-CHMM and Discrete Hidden Markov Model-DHMM) combined to other techniques Hough Transformation related approaches combined to other techniques | 2 1 | 13% 6% |
|  |  | Regression Based Models combined to other techniques | 1 | 6% |
|  |  | Novel Image Recognition Technique | 1 | 6% |

- FINAL REMARKS

Machine Perception has more studies with practical results. Considering the other components, few studies with practical results from real deployments were found. Most of the papers presented preliminary results. In fact, some papers start with a promise and ﬁnish with more promises. Considering only 24% of the total papers considered in this study were published by journals, it is possible to conclude the ﬁeld is not mature yet.

Some similar issues were studied in more than one paper about Machine Perception, and distinct techniques were applied to address them (for example, ANN and SVM applied to similar issues as well issues as well distinct techniques applied to the topic cyber-attack). Considering some of those techniques have different working mechanisms, that fact can be an important ﬁnding for the safety of autonomous cars as regards the need of redundant components. Similar issues being addressed by different techniques were not identiﬁed for Machine Control.

The papers related to Navigation and Control also reported positive and promising results, although the level of maturity of the achievements are clearly much lower than the sensors and perception as well as far from what would be expected for an autonomous vehicle considering the potential hazardous situations it may face. In fact, most of the results presented are preliminary.

Only few of the studies related to system described practical results from real deployments. The papers proposing conceptual models and frameworks bring important contributions, but they are mostly not tested in real set-ups. There is thus a lack of reported results from models and frameworks that could build the foundation of AVs safety. Also, few human-in-the-loop studies had practical results from real deployments. However, they seem to be one of the most important topics seeing that there will be more semi- autonomous cars than fully autonomous ones for a while, and they will co-exist. The human factor will thus be an important variable in the system to be considered not only as the impacted side of the safety, but as one of the sources of interactions inﬂuencing the safety levels. The topic requires

multidisciplinary studies involving ﬁelds beyond engineering and computer science, such as neurosciences. This shows the ﬁeld is not mature yet.

The amplitude and range of the reported future researches in the papers reviewed suggest that there is an empty space for new research into this ﬁeld. For example, only few studies were found about the three topics positioning AI as a potential source of negative impact on safety - Fault Forecasting, Ethics and Policies, and Dependability and Trust. When combined to the other ﬁndings reported by the present study, it conﬁrms the impressions formed during an exploratory research of the literature [1]. It reinforces the perception that the ﬁeld of AI and AV is not heavily inﬂuenced by the safety engineering culture yet. In fact, the studies published about this current topic seem to be more driven by computation-related domains, with no tradition regarding safety culture, than other ﬁelds that are much more connected to safety in critical systems [1].

Additional research is necessary for most of the studies reviewed. They need to be extended to be tested in simulated or real set-ups, new and broader scenarios, with new and more data, and consider experimental designs whereby the results from the proposed approach are compared to benchmarks and alternative techniques. Many AI techniques have achieved impressive results; however, it is still arguable whether the error rates are suitable for real deployments in AVs under the light of a (missing) hazard analysis. Therefore, additional studies with improvements in those techniques are required. Finally, a stronger inﬂuence of safety engineering on most of the studies would beneﬁt the ﬁeld.

A research agenda must consider a serious safety agenda for future studies, at system-level, component-level and AI technique-level. In fact, there are some topics related to safety concerns over AVs, which are critical-path to the development of the ﬁeld. Some of the suggested topics are related to the challenges with validating machine-learning- based systems to the ultra-dependable levels required for AVs; wider and deeper studies about human-machine collaboration in the context of AVs; autonomy assurance and trust in AVs; ethical and moral decisions in the context of AVs; among other topics, from

Validating machine-learning-based systems to the ultra- dependable levels required for AVs; and autonomy assurance and trust in AVs seem to be the holy grail towards a fully autonomous AV - SAE level 5 . They are also key topics for the Safety Certiﬁcation of non-deterministic control systems. In those contexts, there are many gaps to be ﬁlled by future researches, such as AVs software testing, Fault Injection Testing for AI on AVs, Failure Modes and Effects Analysis (FMEA) for AI on AVs, AI safeguards for AVs, AI safety envelopes for AVs, AI redundancy for AVs (many possible approaches, such as a hybrid connectionist and symbolic architecture using causal inference), explainable AI for AVs, AI fault forecasting. Finally, studies on V2X communication can help autonomy assurance by providing channels for hardware and software redundancy. Human-machine collaboration in the context of AVs is another key topic with special impact on the semi-autonomous vehicles (SAE levels 1 to 4). Investigations on the best way humans and AVs can interact during normal

operations and facing hazardous situations are needed to meet the adequate safety requirements the semi-autonomous vehicles must have. Those studies must consider hybrid collective decision-making systems to enable humans and machines to work together and to agree on common decisions, as well as how to deal with the lack of agreement in some situations.

Also, there is another important discussion arising in the context of human-machine collaboration that must be investigated. On the one hand, there are reports about advanced driver assistant technologies that failed (such as Tesla Autopilot) and the driver was not able to react in time to avoid the accident. They ended-up in life losses and property damages. On the other hand, there are reports about situations in which the advanced driver assistant technologies saved the drivers’ life by automatically taking the driver suffering a heart attack to the hospital; fully controlling the car with a drunk driver sleeping; and using a defensive lane change

TABLE VII

HYBRID AI APPROACHES X TOPIC

# Main AI Technique	Topic	AI Techniques	Reference

Conceptual Model and Framework

HoughTransforms, HoughLines, LocalMaximaFinder, Kalman filters and Convolutional Neural Network (CNN)

[60]

Fault Prevention	KNN, SVM Regression (SMO), ANN	[52] CBR, ANN, fuzzy logic, Nearest-Neighbor Retrieval

Artificial Neural Network

Navigation and Control

Sensors and

Algorithm, Basic AI Path Planning algorithms such as A* and

D*

ANN combined to Genetic Algorithm - Neuroevolution of Augmenting Topologies (NEAT)

ANNs, AdaBoost, SVM, Hidden Markov Models (HMMs), CRFs

[39]

[47]

[30]

Perception

Clustering algorithm k- mean, ANN	[31]

HOG, SVM, PCA, ANN	[29]

Hidden Markov Based Models

Navigation and Control Sensors and Perception

GMM, Continuous Hidden Markov Model (CHMM), Discrete Hidden Markov Model (DHMM)

HMM, Viterbi algorithm, Adaboost trained Haar-like feature detector

[45]

[36]

Hough Transformation	Navigation and

Control

Haar Feature Based Cascade Classifier, Canny edge detection and Hough line transformation

[38]

Novel Image Recognition Technique Regression Based Models

Sensors and Perception Navigation and Control

Combination of mathematical techniques	[34]

(DRF) and Linear Regression (LR)	[76]

Haar, HOG, LBP, Chanel features, SVM	[37]

Support Vector Machine (SVM)

Sensors and Perception

k-Nearest Neighbours (kNN), Naïve Bayes classifier (NBC), SVM

[27]

Principal component analysis network (PCANet), SVM	[35]

SVM, HOG	[75]

maneuver to avoid being hit by a truck changing its lane. Some players in the industry are pushing the automation evolution steps towards full automation by requiring the human driver to be a backup to the automated driver. Other players in the industry believe the automated driver must be a backup to the human driver. It looks like the second approach can be a

smoother and safer path towards a SAE level-5 automation.

Immersive environments for training and testing AVs represent another research trend. As the underlying technologies supporting AVs development evolve, higher automation-levels become possible. Considering the potential hazards until the AVs are well trained  and ﬁne-tuned, the

immersive technologies are becoming an important tool to support the development, training and tests of fully autonomous machines. Another broad topic requiring further research is related to ethical and moral decisions in AVs. Some studies only mention issues related to moral dilemmas while others provide some simple experiments involving simulated environments and/or human interviews. However, they misinterpret important concepts and bring the discussions around the decisions AVs must make when life losses are involved, besides the moral and ethical perceptions from the human perspective. All of them miss important points such as statistical considerations and the societal result. In other words, the discussions are not deep enough as regards situations such as whether an AV should hit an old man or a child, while a true safe machine control should consider all the probabilities involved and select the one that minimizes the chances of life losses instead of just picking an option. For example, the system must consider small signals, such as which of the potential victims is paying attention to the approaching AV and what would their potential reaction be and effectiveness of it based on the age and other metrics, as well, considering the multiple scenarios, and the conﬁguration of each, such as speed, region of the car hitting which region of each victim, the potential damages and the severity of the damages considering the estimated weight and overall physical condition, to decide based on the minimization of chances of life losses. This approach will result into higher safety levels for society. Finally, only 1 paper about autonomous truck was found. Considering some speciﬁcities of autonomous truck and its risks, at least a few more studies about the topic could be expected.

REFERENCES

- A. M. Nascimento, L. F. Vismari, P. S. Cugnasca, J. B. Camargo, J. R. de Almeida, R. Inam, E. Fersman, A. Hata, and M. V. Marquezini. Concerns on the differences between ai and system safety mindsets impacting autonomous vehicles safety. In Barbara Gallina, Amund Skavhaug, Erwin Schoitsch, and Friedemann Bitsch, editors, Computer Safety, Reliability, and Security, pages 481–486, Cham, 2018. Springer International Publishing.

- Eusebio Scornavacca, Stuart J. Barnes, and Sid L. Huff. Mobile business research published in 2000-2004: Emergence, current status, and future opportunities. CAIS, 17:28, 2006.

- Oswald Jones and Caroline Gatrell. Editorial: The future of writing and reviewing for ijmr. International Journal of Management Reviews, 16(3):249–264, 2014.

- Jane Webster and Richard T. Watson. Analyzing the past to prepare for the future: Writing a literature review. MIS Q., 26(2):xiii–xxiii, June 2002.

- Sara Abbaspour Asadollah, Daniel Sundmark, Sigrid Eldh, Hans HansS son, and Wasif Afzal. 10 years of research on debugging concurrent and multicore software: A systematic mapping study. Software Quality Journal, 25(1):49–82, March 2017.

- Kai Petersen, Robert Feldt, Shahid Mujtaba, and Michael Mattsson.Systematic mapping studies in software engineering. In Proceedings of the 12th International Conference on Evaluation and Assessment in Software Engineering, EASE’08, pages 68–77, Swindon, UK, 2008. BCS Learning & Development Ltd.

- Yair Levy and Timothy J. Ellis. A systems approach to conduct an effective literature review in support of information systems research. Informing Science: The International Journal of an Emerging TransdisI cipline, 9:181–212, January 2006.

- Jan vom Brocke, Alexander Simons, Bjoern Niehaves, Bjorn Niehaves, Kai Riemer, Ralf Plattfaut, and Anne Cleven. Reconstructing the giant: On the importance of rigour in documenting the literature search process. In Susan Newell, Edgar Whitley, Nancy Pouloudi, Jonathan Wareham, and Lars Mathiassen, editors, Information systems in a globalising world

: challenges, ethics and practices ; ECIS 2009, 17th European Conference on Information Systems, pages 2206–2217, Verona, June 2009. Universita` di Verona, Facolta` di Economia, Departimento de Economia Aziendale.

- Lars Mathiassen, Mike Chiasson, and Matt Germonprez. Style composition in action research publication. MIS Q., 36(2):347–363, June 2012.

- Barbara Kitchenham. Procedures for performing systematic reviews. Keele university. technical report tr/se-0401, Department of Computer Science, Keele University, UK, 2004.

- Eusebio Scornavacca Jr. and Stuart J. Barnes. M-banking services in japan: a strategic perspective. Int. J. Mob. Commun., 2(1):51–66, May 2004.

- Jason Brownlee. A tour of machine learning algorithms. Machine Learning Mastery, 2013.

- Abdullah Gani, Aisha Siddiqa, Shahaboddin Shamshirband, and Fariza Hanum. A survey on indexing techniques for big data: taxonomy and performance evaluation. Knowledge and Information Systems, 46(2):241–284, Feb 2016.

- Mark Hall, Eibe Frank, Geoffrey Holmes, Bernhard Pfahringer, Peter Reutemann, and Ian H. Witten. The weka data mining software: An update. SIGKDD Explor. Newsl., 11(1):10–18, November 2009.

- Baharum Baharudin, Lam Hong Lee, Khairullah Khan, and Aurangzeb Khan. A review of machine learning algorithms for text-documents classiﬁcation. Journal of Advances in Information Technology, 1, 02 2010.

- Stuart Russell and Peter Norvig. Artiﬁcial Intelligence: A Modern Approach. Prentice Hall Press, Upper Saddle River, NJ, USA, 3rd edition, 2009.

- Ian H. Witten, Eibe Frank, and Mark A. Hall. Data Mining: Practical Machine Learning Tools and Techniques. Morgan Kaufmann Publishers Inc., San Francisco, CA, USA, 3rd edition, 2011.

- K. Yoneda, A. Kuramoto, and N. Suganuma. Convolutional neural network based vehicle turn signal recognition. In 2017 International Conference on Intelligent Informatics and Biomedical Sciences (ICIC IBMS), pages 204–205, Nov 2017.

- Robert L Simpson. Strategic Computing Computer Vision: Taking Image Understanding To The Next Plateau. In Image Understanding and the Man-Machine Interface, volume 758, pages 52–60. International Society for Optics and Photonics, 1987.

- B. Kihei, J. A. Copeland, and Y. Chang. Automotive doppler sensing: The doppler proﬁle with machine learning in vehicle-to-vehicle networks for road safety. In 2017 IEEE 18th International Workshop on Signal Processing Advances in Wireless Communications (SPAWC), pages 1– 5, July 2017.

- Qing Li, Nanning Zheng, and Hong Cheng. Springrobot: a prototype autonomous vehicle and its algorithms for lane detection. IEEE Transactions on Intelligent Transportation Systems, 5(4):300–308, Dec 2004.

- Wilfried Woeber, Martin Kefer, W Kubinger, and D Szuegyi. Evaluation of daylight and thermal infra-red based detection for platooning vehicles. 23rd DAAAM International Symposium on Intelligent Manufacturing and Automation 2012, 2:719–722, 01 2012.

- R. Dominguez, E. Onieva, J. Alonso, J. Villagra, and C. Gonzlez. Lidar based perception solution for autonomous vehicles. In 2011 11th In2 ternational Conference on Intelligent Systems Design and Applications, pages 790–795, Nov 2011.

- Gianni Vernazza Daniele D. Giusto, Carlo S. Regazzoni. Multisensorial vision for autonomous vehicle driving. volume 1095, pages 1095 – 1095

– 8, 1989.

- M. Allodi, A. Broggi, D. Giaquinto, M. Patander, and A. Prioletti. Machine learning in tracking associations with stereo vision and lidar observations for an autonomous vehicle. In 2016 IEEE Intelligent Vehicles Symposium (IV), pages 648–653, June 2016.

- Alex Dominguez-Sanchez, Sergio Orts-Escolano, and Miguel Cazorla. Recognizing pedestrian direction using convolutional neural networks. In Ignacio Rojas, Gonzalo Joya, and Andreu Catala, editors, Advances in Computational Intelligence, pages 235–245, Cham, 2017. Springer International Publishing.

- Navarro Pedro J.; Fernndez C.; Borraz R.; and Alonso D. A machine learning approach to pedestrian detection for autonomous vehicles using high-deﬁnition 3d range data. Sensors (Basel), 17(1), 2016.

- Julian Bock, Jens Kotte, Till Beemelmanns, and Markus Klsges. Self- learning trajectory prediction with recurrent neural networks at intelligent intersections. In Proceedings of the 3rd International Conference on Vehicle Technology and Intelligent Transport Systems - Volume 1: VEHITS, pages 346–351. INSTICC, SciTePress, 2017.

- J. Jeon, S. Hwang, and H. Moon. Monocular vision-based object recognition for autonomous vehicle driving in a real driving environment. In 2016 13th International Conference on Ubiquitous Robots and Ambient Intelligence (URAI), pages 393–399, Aug 2016.

- D. Habermann, C. E. O. Vido, F. S. Osrio, and F. Ramos. Road junction detection from 3d point clouds. In 2016 International Joint Conference on Neural Networks (IJCNN), pages 4934–4940, July 2016.

- F. Ebadi and M. Norouzi. Road terrain detection and classiﬁcation algorithm based on the color feature extraction. In 2017 Artiﬁcial Intelligence and Robotics (IRANOPEN), pages 139–146, April 2017.

- S. Hamdi, H. Faiedh, C. Souani, and K. Besbes. Road signs classiﬁcation by ann for real-time implementation. In 2017 International Conference on Control, Automation and Diagnosis (ICCAD), pages 328–332, Jan 2017.

- Eshed Ohn-Bar and Mohan Manubhai Trivedi. Are all objects equal? Deep spatio-temporal importance prediction in driving videos. Pattern Recognition, 64:425 – 436, 2017.

- W. Devapriya, C. N. K. Babu, and T. Srihari. Advance driver assistance system (adas) - speed bump detection. In 2015 IEEE International Conference on Computational Intelligence and Computing Research (ICCIC), pages 1–6, Dec 2015.

- Z. Chen and X. Huang. Accurate and reliable detection of trafﬁc lights using multiclass learning and multiobject tracking. IEEE Intelligent Transportation Systems Magazine, 8(4):28–42, winter 2016.

- N. Miller, M. A. Thomas, J. A. Eichel, and A. Mishra. A hidden markov model for vehicle detection and counting. In 2015 12th Conference on Computer and Robot Vision, pages 269–276, June 2015.

- Antonio M. Lopez, Gabriel Villalonga, Laura Sellart, Germn Ros, David Vzquez, Jiaolong Xu, Javier Marn, and Azadeh Mozafari. Training my car to see using virtual worlds. Image and Vision Computing, 68:102 – 118, 2017. Automotive Vision: Challenges, Trends, Technologies and Systems for Vision-Based Intelligent Vehicles.

- M. R. T. Hossai, M. A. Shahjalal, and N. F. Nuri. Design of an iot based autonomous vehicle with the aid of computer vision. In 2017 International Conference on Electrical, Computer and Communication Engineering (ECCE), pages 752–756, Feb 2017.

- B. M. Elbagoury, A. M. Salem, and L. Vladareanu. Intelligent adaptive precrash control for autonmous vehicle agents (cbr engine amp; hybrid a path planner). In 2016 International Conference on Advanced Mechatronic Systems (ICAMechS), pages 429–436, Nov 2016.

- Jorge Godoy, Joshu Prez, Enrique Onieva, Jorge Villagr, Vicente Milans, and Rodolfo Haber. A driverless vehicle demonstration on motorways and in urban environments. Transport, 30(3):253–263, 2015.

- Ondrej Linda, Todd Vollmer, and Milos Manic. Svm-inspired dynamic safe navigation using convex hull construction. 2009 4th IEEE Confer2 ence on Industrial Electronics and Applications, ICIEA 2009, 05 2009.

- I Zohdy and Hesham Rakha. Optimizing driverless vehicles at intersections. pages AM–00071, 01 2012.

- Mikael Asplund, Atif Manzoor, Melanie´ Bouroche, Siobhan` Clarke, and Vinny Cahill. A formal approach to autonomous vehicle coordination. In Dimitra Giannakopoulou and Dominique Mery´, editors, FM 2012: Formal Methods, pages 52–67, Berlin, Heidelberg, 2012. Springer Berlin Heidelberg.

- Erwin Jose Lopez Pulgarin, Guido Herrmann, and Ute Leonards. Drivers’ manoeuvre classiﬁcation for safe hri. In Yang Gao, Saber FalD lah, Yaochu Jin, and Constantina Lekakou, editors, Towards Autonomous Robotic Systems, pages 475–483, Cham, 2017. Springer International Publishing.

- Ge, F., Moore, W., & Antolovich, M. (2015, November). Learning from demonstration using gmm, CHMM and DHMM: A comparison. In Australasian Joint Conference on Artificial Intelligence (pp. 204-217). Springer, Cham.

- I. H. Zohdy, R. K. Kamalanathsharma, and H. Rakha. Intersection management for autonomous vehicles using icacc. In 2012 15th International IEEE Conference on Intelligent Transportation Systems, pages 1109–1114, Sep. 2012.

- Francisco Gallego, Faraon´ Llorens, Mar Pujol, and Ramon´ Rizo. Learning human-level ai abilities to drive racing cars. In Proceedings of the 2005 Conference on Artiﬁcial Intelligence Research and Development, pages 33–40, Amsterdam, The Netherlands, The Netherlands, 2005. IOS Press.

- Citlalli Gamez´ Serna, Alexandre Lombard, Yassine Ruichek, and Abdeljalil Abbas-Turki. Gps-based curve estimation for an adaptive pure pursuit algorithm. In Grigori Sidorov and Oscar Herrera-Alcantara,´ editors, Advances in Computational Intelligence, pages 497–511, Cham, 2017. Springer International Publishing.

- J.E. Naranjo, F. Jimnez, O. Gmez, and J.G. Zato. Low level control layer deﬁnition for autonomous vehicles based on fuzzy logic. Intelligent Automation & Soft Computing, 18(4):333–348, 2012.

- Algirdas Avizienis, Jean-Claude Laprie, Brian Randell, and Carl Landwehr. Basic concepts and taxonomy of dependable and secure computing. IEEE Trans. Dependable Secur. Comput., 1(1):11–33, January 2004.

- P. Sharma, Hong Liu, Honggang Wang, and Shelley Zhang. Securing wireless communications of connected vehicles with artiﬁcial intelliw gence. In 2017 IEEE International Symposium on Technologies for Homeland Security (HST), pages 1–7, April 2017.

- Mostafa Anwar Taie, Eman Magdy Moawad, Mohammed Diab, and Mohamed ElHelw. Remote diagnosis, maintenance and prognosis for advanced driver assistance systems using machine learning algorithms. SAE Int. J. Passeng. Cars Electron. Electr. Syst., 9:114–122, 04 2016.

- Y. Seo, J. Kim, and R. Rajkumar. Predicting dynamic computational workload of a self-driving car. In 2014 IEEE International Conference on Systems, Man, and Cybernetics (SMC), pages 3030–3035, Oct 2014.

- Shirin Noei, Arman Sargolzaei, Alireza Abbaspour, and Kang Yen. A decision support system for improving resiliency of cooperative adaptive cruise control systems. Procedia Computer Science, 95:489 – 496, 2016. Complex Adaptive Systems Los Angeles, CA November 2-4, 2016.

- Newman R.M. Gaura, E. Microsensors, arrays and automatic diagnosis of sensor faults. In Technical Proceedings of the 2003 Nanotechnology Conference and Trade Show, volume 1, pages 276 – 279. TechConnect Briefs, February 2003.

- Quanzhen Guan, Hong Bao, and Zuxing Xuan. The research of prediction model on intelligent vehicle based on driver’s perception. Cluster Computing, 20(4):2967–2979, Dec 2017.

- Ata M. Khan. Design of real-time transition from driving assistance to automation function: Bayesian artiﬁcial intelligence approach. In Jan Fischer-Wolfarth and Gereon Meyer, editors, Advanced Microsystems for Automotive Applications 2014, pages 39–48, Cham, 2014. Springer International Publishing.

- Hamidur Rahman, Shahina Begum, and Mobyen Uddin Ahmed. Driver monitoring in the context of autonomous vehicle. In The 13th ScandiT navian Conference on Artiﬁcial Intelligence, November 2015.

- Ata Khan. Cognitive vehicle design guided by human factors and supported by bayesian artiﬁcial intelligence. In Neville A Stanton, editor, Advances in Human Aspects of Transportation, pages 362–372, Cham, 2018. Springer International Publishing.

- P. K. Gurudatt and V. Umesh. Novel architecture for cloud based next gen vehicle platform transition from today to 2025. In 2017 IEEE International Conference on Consumer Electronics-Asia (ICCE-Asia), pages 151–155, Oct 2017.

- Silviu Ionita. Autonomous vehicles: from paradigms to technology. IOP Conference Series: Materials Science and Engineering, 252(1):012098, 2017.

- A. Rakotonirainy, O. Orﬁla, and D. Gruyer. Reducing driver’s behavioral uncertainties using an interdisciplinary approach: Convergence of quantiﬁed self, automated vehicles, internet of things and artiﬁcial intelligence. IFAC-PapersOnLine, 49(32):78 – 82, 2016. Cyber-Physical & Human-Systems CPHS 2016.

- Akihiro Okuno, Atsushi Kutami, and Kenji Fujita. Towards autonomous cruising on highways. 08 1990.

- Philip Koopman. Challenges in Autonomous Vehicle Validation: Keynote Presentation Abstract. In Proceedings of the 1st International Workshop on Safe Control of Connected and Autonomous Vehicles, page 3. ACM, 2017.

- D. Meltz and H. Guterman. Robil israeli program for research and development of autonomous ugv: Performance evaluation methodology. In 2016 IEEE International Conference on the Science of Electrical Engineering (ICSEE), pages 1–5, Nov 2016.

- C. Wolschke, T. Kuhn, D. Rombach, and P. Liggesmeyer. Observation based creation of minimal test suites for autonomous vehicles. In 2017

IEEE International Symposium on Software Reliability Engineering Workshops (ISSREW), pages 294–301, Oct 2017.

- P. Koopman and M. Wagner. Autonomous vehicle safety: An interdisciplinary challenge. IEEE Intelligent Transportation Systems Magazine, 9(1):90–96, Spring 2017.

- M. Bahram, Z. Ghandeharioun, P. Zahn, M. Baur, W. Huber, and F. Busch. Microscopic trafﬁc simulation based evaluation of highly automated driving on highways. In 17th International IEEE Conference on Intelligent Transportation Systems (ITSC), pages 1752–1757, Oct 2014.

- Wesley Kumfer and Richard Burgess. Investigation into the role of rational ethics in crashes of automated vehicles. Transportation Research Record: Journal of the Transportation Research Board, 2489:130–136, 01 2015.

- G. Lugano. Virtual assistants and self-driving cars. In 2017 15th International Conference on ITS Telecommunications (ITST), pages 1– 5, May 2017.

- Ebru Dogan, Raja Chatila, Stephane Chauvier, Katherine Evans, Petria Hadjixenophontos, and Jerome Perrin. Ethics in the design of automated vehicles: The avethics project. In EDIA@ECAI, 2016.

- John Tasioulas Kristen Brent Venable Brian Williams Joshua Greene, Francesca Rossi. Embedding ethical principles in collective decision support systems. In Proceedings of the Thirtieth AAAI Conference on Artiﬁcial Intelligence (AAAI-16), 2016.

- Rowan McAllister, Yarin Gal, Alex Kendall, Mark van der Wilk, Amar Shah, Roberto Cipolla, and Adrian Weller. Concrete problems for autonomous vehicle safety: Advantages of bayesian deep learning. In

Proceedings of  the Twenty-Sixth International Joint Conference on

Artiﬁcial Intelligence, IJCAI-17, pages 4745–4753, 2017.

- R.; Lacher, A.; Grabowski and S. Cook. Autonomy, trust, and transportation. In AAAI Spring Symposium Series, 2014.

- Keisuke Kazama, Yasuhiro Akagi, Pongsathorn Raksincharoensak, and Hiroshi Mouri. Fundamental study on road detection method using multi-layered distance data with hog and svm. Journal of robotics and mechatronics, 28(6):870–877, dec 2016.

- H. A. Najada and I. Mahgoub. Autonomous vehicles safe-optimal trajectory selection based on big data analysis and predeﬁned user preferences. In 2016 IEEE 7th Annual Ubiquitous Computing, Electronics Mobile Communication Conference (UEMCON), pages 1– 6, Oct 2016.

- Mohammad Ali Abam and Mark de Berg. Kinetic sorting and kinetic convex hulls. In Proceedings of the Twenty-ﬁrst Annual Symposium on Computational Geometry, SCG ’05, pages 190–197, New York, NY, USA, 2005. ACM.

- Jerry Engelman Charles Green Charles Kawashima Steve Kiger Danil Prokhorov Levasseur Tellis Barbara Wendling Andy Christensen, AnP drew Cunningham. Key considerations in the development of driving automation systems. Gothenburg, Sweden, 2015.

- Neil R. Storey. Safety Critical Computer Systems. Addison-Wesley Longman Publishing Co., Inc., Boston, MA, USA, 1996.

- Salvendy G, editor. Handbook of human factors and ergonomics. John Wiley & Sons; 2012 May 24

- Norman, D. (2013). The design of everyday things: Revised and expanded edition. Basic books.

APPENDIX

# Technique (CT.3)

TABLE VIII CT.3 X CT.4

# Hits	Papers	Addressed Problem (CT.4)	References

AV Validation; Challenge with validating machine-learning based systems to the ultra-dependable levels required for autonomous vehicle; Coexistence Human Machine Controller; Driving Car Tasks Classiﬁcation; Lack of efﬁcient Safety Performance Veriﬁcation technique when AI/ML is used; Crash assignment, especially between

General AI/ML	14	24%

Artiﬁcial

automated vehicles and non-automated vehicles; Reducing the uncertainty of a driver behavior prediction model; Integration between automatic vehicle and human driver; How the vehicle autonomy technology can be used to beneﬁt car drivers and t o improve highway driving safety by a concept of an autonomous highway vehicle; AV decisions in complex dilemmas as a social agent; Hybrid (humans and machines) collective decision making systems (work together and agree on common decisions); Autonomy assurance and trust (CERTIFICATION PROA CESS) in Automated Transportation Systems; Evaluating the impacts of the number of highly automated vehicles on future trafﬁc safety and trafﬁc ﬂow; Enhancing Image Understanding.

Vehicle Cyber Attack; Turn Signal Recognition; Pre-crash issues of Intelligent Control of autonomous vehicles robot; Real-time   Road Sign Classiﬁcation; Road Terrain detection; Spatio-temporal situation awareness; Pedestrian detection and movement direction  recognition;

[64],[67],

[70],[61],

[65],[69],

[62],[58],

[63],[71],

[72],[74],

[68],[19]

[39],[18],

[32],[31],

Neural Networks (ANN)

Support Vector Machine

(SVM)

13	22%

10	17%

Pedestrian Trajectory Prediction; Road junction detection; Early faults or malfunction detection; Prediction of advanced driver assistance systems (ADAS) remaining useful life (RUL) for the prognosis of ADAS safety critical components; Road environmental recognition and various objects detection in real driving conditions; Human and Machine Driver Co-existence;

Road Detection; Robot maneuvers too close to an obstaR cle; Road environmental recognition and various object detection in real driving conditions; Drivers maneuver classiﬁcation; Trafﬁc Light Detection;

[33],[26],

[28],[30],

[55],[52],

[29],[60]

[75],[41],

[29],[20],

[44],[35],

| Bayesian Artiﬁcial | 4 | 7% |
| --- | --- | --- |
| Intelligence |  |  |
| Fuzzy Logic | 5 | 8% |
| Hidden Markov | 4 | 7% |

Based Models

Estimation Filters	4	7% Nearest

Neighbour-Based	4	7%

Algorithm

Prediction of adc vanced driver assistance systems (ADAS) remaining useful life (RUL) for the prognosis of ADAS safety critical components Pedestrian   Detection;	How to ”automate” manual annotation for images to train visual perception for AVs Road junction detection; Collision avoidance when no action is taken by driver; Safety, interpretability,	and   compliance;	Pedestrian	Detection; Design of driving assistance system;

PreCrash problem of Intelligent Control of autonomous vehicles robot; Driverless car 100 km experiment Cyber Attack in   V2X;   Manage low level vehicle actuators (steering throttle and brake); Road and Obstacle Detection;

Vehicle Detection and Counting; Road junction detection; Learn from Demonstration;

Human and Machine Driver Co-existence; Securing connected vehicles against Denial of Service (DoS) attack; Reliable and robust obstacles detection;

Pre-crash problem of Intelligent Control of autonomous vehicles robot; Pedestrian Detection; Providing road   safety to connected drivers and connected autonomous vehicles;

Vehicle Detection and Counting; Leading vehicle recogV nition in platooning; Road junction detection;

[52],[27],

[37],[30]

[57],[73],

[27],[59]

[39],[40],

[54],[49],

[23]

[36],[29],

[45]

[60],[51],

[24]

[39],[26],

[19]

[36],[21],

[29]

| Adaptive |  |  |
| --- | --- | --- |
| Boosting | 3 | 5% |
| (AdaBoost) |  |  |
| Ramer-Douglas |  |  |
| Peucker |  |  |
| or Ramer- | 3 | 5% |
| Douglas |  |  |
| algorithm |  |  |

Obstacle clustering and tracking; Path tracking;	[22],[48]

detector

Developing remotecontrolled car with some automation to deal with

| Haar-like |  |  | trafﬁc light | detection, obstacle | avoidance | system and lane detection |
| --- | --- | --- | --- | --- | --- | --- |
| feature | 3 | 5% | system to | be driven from | anywhere | over a secured internet |

connection;   Vehicle Detection and Counting;   How to ”automate”

| Histogram of Oriented Gradient | 3 | 5% |
| --- | --- | --- |
| (HOG) |  |  |
| Hough Transfor- | 3 | 5% |
| mation |  |  |
| Optimiza-tion Heuristics | 3 | 5% |
| Regression- Based | 3 | 5% |
| Models |  |  |
| Principal Componen-ts | 2 | 3% |
| Analysis (PCA) |  |  |
| Canny Edge De- tection | 1 | 2% |
| Algorithm |  |  |
| Case-based reasoning | 1 | 2% |
| (CBR) Channel Features | 1 | 2% |

manual annotation for images to train visual perception for AVs; Road Detection; Road environmental recognition and various objects

detection   in real driving conditions;   How   to ”automate” manual

annotation for images to train visual perception for AVs;

Road Detection; Road environmental recognition and various object detection in real driving conditions; How to ”automate” manual annotation for images to train visual perception for AVs;

Control intersection crossing (all way stop) and op- timization; Autonomous vehicles intersections crossing optimization; Human and Machine Driver Co-existence;

Selective Attention Mechanism; Safe-optimal trajectory selection for autonomous vehicles; Predicts the CPU usage patterns of software tasks running on a self-driving car;

Trafﬁc Light Detection; Road environmental recognition and various object detection in real driving conditions;

Developing remote- controlled car with some   automation   to deal with trafﬁc light detection, obstacle avoidance system and lane detection system to be driven from anywhere over a secured internet connection;

[38],[36],

[37]

[75],[28],

[37]

[60],[38],

[20]

[46],[42],

[60]

[56],[79],

[53]

[35],[28]

[38]

Pre-crash problem of Intelligent Control of autonomous vehicles robot;	[39]

How to ”automate” manual   annotation   for images   to train	visual perception for AVs;

[37]

Clustering Algo- rithm k- mean

Complex Decision Trees (CDT)

Conditional

1	2%	Road Terrain detection;	[30]

1	2%	Providing road safety to connected drivers and connected   autonomous	[19] vehicles;

| Random | 1 | 2% | Road junction detection; | [29] |
| --- | --- | --- | --- | --- |
| Fields (CRFs) |  |  |  |  |
| Distributed |  |  |  |  |
| Random Forest | 1 | 2% | Safe-optimal trajectory selection for autonomous vehicle; | [79] |
| (DRF) |  |  |  |  |
| Gaussian |  |  |  |  |
| Mixture Model | 1 | 2% | Learn from Demonstration; | [45] |
| (GMM) |  |  |  |  |
| Linear |  |  |  |  |
| Temporal | 1 | 2% | AV Test; | [66] |
| Logic (LTL) |  |  |  |  |

Local Binary Patterns (LBP)

Neuroevo-lution of	Augmen-

1	2%	How to ”automate” manual annotation  for images to train	visual perception for AVs;

Learning and simulation of the Human-Level decisions   involved in

[37]

ting Topologies (NEAT)

Novel

1	2%

driving a racing car;	[47]

Image Recogni-tion Technique Path Planning Al- gorithms (A* and D*) Satisﬁability Modulo Theories (SMT) Solver Viterbi Algorithm

1	2%	Speed bump detection;	[34]

1	2%	PreCrash problem of Intelligent Control of autonomous vehicles robot;	[39]

1	2%	How to prove the correctness of an algorithm for Vehicle Coordination;	[43]

1	2%	Vehicle Detection and Counting;	[36]

TABLE IX

FINDINGS ON PAPERS ORIENTED BY THE DISCUSSION

Issue

Suggested Approach

AI

# Technique

Findings	Reference

DDT System Model Component - System

Human and Machine Driver Co-existence

Continuously monitor the driving behavior of the neighboring vehicles, sensor behavior and processor behavior of the ego vehicle regardless of the vehicle being autonomous or not

ANN (Hybrid)

The authors have foreseen and proposed

the solutions for future problems, which would occur while the autonomous vehicles are a part of driving. All the three described architectures have addressed the safety related problems. The architectures are based on the availability of the resources for vehicles. The first two architectures address the safety failure due to the human ignorance and autonomous vehicle behavior. Third architecture addresses the way of securing the failed vehicle due to system failure. All the architectures rely highly on the efficient connectivity and

[60]

Investigate		three under-explored themes	for			AV research:		safety, interpretability, and compliance

End-to-end Bayesian deep	learning architecture to propagate uncertainty throughout the AV framework. In this case, standard deep learning makes hard predictions, whereas Bayesian deep learning outputs probabilistic predictions accounting for each model’s ignorance about the world.

ANN (Hybrid)

computer vision algorithms.

3 critical themes for a smooth adoption of AV systems by society were highlighted. Hard decisions are dangerous. Soft (uncertain) classifications should be propagated through to the decision layer. This enables the AV to act more cautiously in the event of greater uncertainty. We also discussed the themes. Authors argument interpretability and compliance as ways to build trust and mitigate fears which passengers might otherwise reasonably have about unfamiliar black-box AV systems. Also, they discussed about the importance of clear metrics to evaluate each component’s probabilistic output based on their ultimate effect on the vehicle’s performance.

[73]

AV Validation	Safety Envelopes	General

AI/ML

No findings, only suggested approaches.	[64]

Challenge		with validating machine- learning	based systems to the ultra- dependable levels required			for autonomous vehicle

Coexistence Human Machine Controller

Driving Car Tasks Classification

Lack of efficient Safety Performance

Safety	certification strategy addressing the cross-disciplinary concerns	of			safety engineering,		hardware reliability,			software

validation, robotics, security, testing, human- computer interaction, social acceptance, and a viable legal framework.

In-car	Virtual Assistants

Classification of the tasks that take place during the driving of the vehicle and its modeling from the perspective of traditional	control engineering and artificial intelligence

Methodology	to generate an estimation of

General AI/ML

General AI/ML

General AI/ML

General AI/ML

The paper only points out the challenge with validating machine-learning based systems to the ultra-dependable levels required for autonomous vehicle fleets, and how that challenge relates to a number of other areas. It does not provide any particular finding.

It is too early to assess whether carmakers' optimistic vision of in-car virtual assistants will match users' experience or follow a similar destiny of "Clippy the Paperclip" from Microsoft Word. Proprietary systems are predominant and there is no common framework addressing ethical principles, liability, data protection, privacy and security on many of the technologies associated to AVs. Due to these shortcomings, allowing a fully autonomous approach could generate more drawbacks than benefits. At least in a first phase, it seems more appropriate to apply AI-based virtual assistants to autonomously execute tasks and take decisions (i.e. replace humans) on safety-related functions only, in line with the requirements defined by international standards.

The major issues realted to safety and and the efforts to make sure the technologies involved are robust are discussed: test the safety of the ADAS, standardization, the development of models and algorithms, the appropriate constructing solutions for implementation, and ethical issues. No specific findings are presented.

Detailed methodology was proposed to deal with the issue by means of statistical

[67]

[70]

[61]

[65]

Verification technique	when AI/ML is used

Crash assignment, especially between automated vehicles and nonautomated vehicles

Reduce	the

uncertainty of a driver behaviour prediction model

How	vehicle autonomy technology can be used to benefit car drivers and also to

probability of fatal mishap	of		an autonomous		UGV navigation algorithm based on Statistical Testing in a MonteCarlo manner in a Simulated Environment

The integration of three ethical theories— utilitarianism, respect for persons, and virtue ethics—with vehicle automation is used in a simple crash scenario where an automated vehicle must choose between three crash types on the basis of a randomly	assigned ethical theory to understand the outcomes of distinct ethical frameworks

Proposes a Data Analysis Framework to exploit AI, quantified self, internet of things and automated driving to build a computational driver		behavioural model aming to reduce the uncertainty of a driver			behaviour prediction model and be used monitor, predict and	control	a transportation system.

General AI/ML

General AI/ML

General

testing via simulation. Demonstration was still a work in process.

The results of the experiment indicated that in specific crash scenarios, utilitarian ethics may reduce the total number of fatalities that result from automated vehicle crashes, although other ethical systems may be useful for developing rules used in machine learning. The experiment demonstrates that understanding rational ethics is crucial for developing safe automated vehicles.

It is very hard to predict due to the fluidity and interactions of the driving factors determining the driver performance.

No specific findings were presented by

[69]

[62]

propose a concept of an	autonomous highway        vehicle

which        improves

highway driving safety

AV    decisions    in

Conceptual discussion

Proposition	of	a framework for an ethics

AI/ML

General

the conceptual discussion	[63]

The ethics of automated vehicles is becoming a major issue from legal, social, and vehicle control perspectives. AV will have	to	make	decisions that might eventually harm an agent and that

complex	dilemmas

as a social agent

policy for the artificial intelligence of an AV

AI/ML

these decisions should not contradict the

interests of the end users or the principal stakeholders. An ethics policy for automated vehicles is needed and the proposed framework (AVEthics) is only the beginning of a long path.

[71]

Humans	and machines will often need to work together and agree on common decisions.

Autonomy assurance and	trust

(certification

Shared moral values and ethical principles

Framework Proposition	for	the

General AI/ML

General

Hybrid	collective	decision-making	[72] systems will be in great need

Authors explored some of the unique challenges that autonomous transportation systems present with regard to traditional certification approaches such being non- deterministic and employing adaptive algorithms. Authors discussed the concept of multiparty trust and how it can be

process)	in

Automated Transportation Systems

Evaluate the impacts of the number of highly automated vehicles on future traffic safety and traffic flow

AV Test

discussion around the topic

Framework		to evaluate the impacts of AV on traffic and the impact of continuous increase in the number of highly	automated vehicles on future traffic safety and traffic flow

Creation of Minimal Test-Suites with Test- cases for the validation of AVs using recordings of traffic situations

AI/ML

General AI/ML

LTL

extended to a framework illustrating the

relationships between disparate roles. Two thought experiments showed that building and

maintaining trust in the perception and judgment of increasingly autonomous systems will be a challenge for the transportation community.

The results of impact evaluation in this study show that the increase in the penetration rate of the highly automated vehicle together with proper adjustment of model parameter may result in considerate improvements of safety in traffic in terms of defined indicators. The developed methodology allows to compare traffic efficiency and safety measures with different penetration rates in various scenarios by means of microscopic traffic simulation.

The process of test-case derivation can be applied was demonstrated. According to the authors, the derivation of test-cases categorizes the recordings automatically and allows test engineers to specify test inputs. The test-case descriptions use the Linear Temporal Logic (LTL) and allow the execution of continuous behaviors, which may also react to the behavior of the tested vehicle. According to the authors, as the traffic recordings can also be used for machine learning algorithms, the contributes to the discussion of their safety certification. They also state the approach is flexible as it can be extended if new traffic situations are supposed to be covered by testing.

[74]

[68]

[66]

DDT System Model Component – Human-in-the-loop

Collision avoidance when no action is taken by driver to avoid the collision

Design of driving assistance system

Real time transition from assisted driving to automated driving under conditions of high probability of a collision if no action is taken to avoid the collision

Discussion about Design considerations are advanced in order to

Bayesian Artificial Intelligence

Bayesian Artificial Intelligence

Systems can be designed to feature collision warnings as well as automated active safety capabilities. The high-level architecture of the Bayesian transition model seems promising. Example scenarios illustrate the function of the real-time transition model.

No specific findings, only discussions about the important considerations to be taken into account when designing AVs

[57]

[59]

Integration between AV and human driver

Develop remote controled car with some automation to deal with traffic light detection, obstace avoidance system and lane detection system to be driven from anywhere over a secured internet connection

Selective   Attention

overcome issues in in- vehicle	telematics systems

un-obstructive human driver	monitoring approaches to ensure they will be able to take over control within short notice

Traffic		Light Detection: Haar Feature Based	Cascade Classifier;			Lane Detection: Canny Edge detection and Hough line transformation was used

General AI/ML

Hough Transformation related approaches combined to other techniques

Regression

One of the most essential parts of the autonomous driving system is to monitor driver’s physical and mental state to avoid unexpected traffic accidents. There are some solutions for non-contact measurement of vital signs, such as HR, RR include laser Doppler, microwave Doppler radar, and thermal imaging. Several AI approaches that have been applied in classifying non-contact physiological sensor signals in different other domains could be possible to investigate in classifying	driver’s	signals. This paper shows that the assessment of non-contact physiological parameters presents a greater challenge and few attempts have been made to adopt it for the driving situation.

Low-cost remote-controlled car prototype with basic automated functions and using basic off-the-shelf components with promising results. It proved cheap and useful prototypes can be built for research. Results with the proposed road detection techniques proved to be highly efficient for a road with clearly visible lane market. Canny Edge detection proved to have low error rate.

Besides, the model is consistent with the famous Weber–Fechner law. The Weber– Fechner law says that all people’s feeling, including visual feeling, auditory feeling

[58]

[38]

Mechanism	Weber–Fechner law

Based Models

and so on all comply with the fact that the feeling is not proportional to the strength of the corresponding physical quantity but proportional to the logarithm of the corresponding physical quantity.

[56]

DDT System Model Component – Machine Control

Its is flexible to be integrated to lower levels of vehicles controller and path planners as (A* & D*). It is an ongoing

Pre-crash problem of AV	Intelligent Control

Learning	and

simulationf	the human-level

AV	Intelligent Adaptive		Control architecture using an hybrid AI architecture: CBR Engine for Adaptive control (high level) + hybrid Case- Based Planner (A* and D* motion planner)

Use Neuroevolution of	Augmenting Topologies (NEAT) and

Artificial Neural Network combined to other techniques

Artificial Neural Network

reasearch. Some limited and embrionary experimental are mentioned and the authors claim present research ideas for different pre-crash scenarios and cases such as intersection safety and some general cases. The paper also discusses approaches to integrate basic kinematics of AVs features and presents future prospective on the possibility of integration of high-level intelligent controller with lower-levels mechanical and kinematics features of vehicles or robotics in general concepts.

Pilots' learning curve is irregular, due to the characteristics of the problem, but presents a good positive tendency which

[39]

[47]

decisions involved in driving a racing car

AV experiment (100 km)

Manage and control low level vehicle actuators (steering throttle and brake)

a online videogame prototype as a test-bed environment. NEAT is a combination of ANN and Genetic Algorithm (GA)

AV following a manually driven car (trailing)

Control schema to manage low level vehicle	actuators (steering throttle and brake) based on fuzzy logic

combined to other techniques

Fuzzy Logic

Fuzzy Logic

leads them to acquire fruitful abilities in just

50 generations with a population of 120 individuals. Pilots easily learn how to turn following soft cruves, but still have big poblems ientifying and steering hard ones, which even make them crash into track limits sometimes. The paper presents individuals habing only 2 output neurons: one for turning and the other one for throttling/braking. Therefore, for this reatively young ANNs, is very dificult to acquire the high-level behaviours that have to completely change the sign of the output of the second neuron, whenevver a sharp curve is near.

The authors state this paper is one of the first communications fully describing the control system and techniques required to perform an experiment with autonomous vehicles on open roads. It introduced a different control approach for controlling autonomous vehicles on urban and motorway environments. A method for online adjustment of the CACC fuzzy controller is described and implemented, coping with the most relevant disturbances and uncertain parameters, such as road slopes, passenger weight, or gear ratio. The experiment successfully proves the capability of the developed system to drive more than one hundred kilometres autonomously. A public demonstration of the described system was conducted in June 2012, comprising a 100-km route through urban and motorway environments. It was able to cope with such gaps as motorway overpasses, traffic signals, etc.The authors stated the tracking results obtained with the CACC system were very precise, with the distance error being kept to less than 1 metre. Likewise, the lateral control was able to maintain the vehicle on the path of the leader with acceptable errors for both scenarios. However, the localization system needs to be improved to allow longer GPS gaps. Also, the presence of a 900-metre- long tunnel forced the deactivation of the autonomous system while the vehicle passed through.

The proposed automatic low-level control system has been defined, implemented and tested in a Citroen C3 testbed vehicle, whose actuators have been automated and can receive control signals from an onboard computer where the soft computing-based control system is running. The preliminary results are confirming the potential of the proposed technique.

[40]

[49]

Control intersection crossing (all way stops) and optimize it

Autonomous vehicles intersections crossing optimization

Path tracking cutting corners	using traditional geometric algorithms

Safe-optimal trajectory	selection for	autonomous vehicle

How to prove the correctness	of		an algorithm		for Vehicle Coordination

Use a simulation to model and simulate the scenario. Developed a heuristic optimization algorithm for driverless vehicles at unsignalized intersections using a multi-agent system.

Proposition of a new tool for optimizing the AVs movements through intersections	- Cooperative Adaptive Cruise Control (CACC)

A curve safety sub- system for path tracking based on the Pure Pursuit algorithm and a dynamic look-ahead distance definition (based on vehicle current speed and lateral error). A sub- system for path tracking where an algorithm that analyzes	GPS

information offline classifies high curvature segments and estimates the ideal speed for each one.

Use Big Data Mining approach for crash prediction and ETA.

Use a distributed coordination protocol and an intersection collision avoidance (ICA) case study + Z3 Theorem prover + Satisfiability Modulo Theories (SMT) solver

Optimization Heuristics

Optimization Heuristics

Ramer- Douglas- Peucker or Ramer- Douglas algorithm

Regression Based Models combined to other techniques

Satisfiability Modulo Theories (SMT) Solver

Although, the proposed research was still in its initial stages, it presented some significant time savings compared to an AWSC intersection control. It showed that by applying the proposed algorithm on only four crossing vehicles, the total delay was reduced by approximately 35 seconds, which is equivalent to a 65 percent reduction in the total intersection delay.

A simulation with one vehicle type and a single intersection was performed. Also, all vehicles in the simulation were assumed to have CACC system to send/receive information and follow speed advices as directed. The preliminary results are promissing and encourage future studies where the authors plan to use simulations with more types of vehicles and a greater number of adjacent intersections. According to the author, the results from this research also warrant studies with regard to incorporating non-CACC vehicles into the system and studies pertaining to tackling unexpected system changes, pedestrian movements etc.

Experimental	results	showed improvements in comfort and safety due to the extracted geometry information and speed control, stabilizing the vehicle and minimizing the lateral error

A Big Data based method and algorithm has been presented to find the safest-optimal trajectory for fully autonomous vehicles. The method proposed relies strongly on the results obtained from Big Data prediction system which predicts accidents, ETA, and clearance time. The algorithms for checking and calculating the optimal trajectory are very lightweight and straightforward. The simulations using the available data are promising.

Paper presented a formalisation of the distributed coordination problem encountered by intelligent vehicles while contending for the same physical resource. It formalised a coordination protocol and an intersection collision avoidance case study in the SMT-lib language and proved system safety using the Z3 theorem prover. The two

[42]

[46]

[48]

[76]

[43]

Robot maneuvers too close to an obstacle increases		the probability	of		an accident. Preventing this is crucial in dynamic environments, where the obstacles, such as other	UAVs,	are moving

Drivers’ manoeuver

classification

to prove correctness and safety of a vehicular coordination problem

SVM-Inspired Dynamic	Safe

Navigation	Using

Convex	Hull

Construction		an algorithm for a fast construction of a maximum	margin between sets of obstacles and its maintenance as the input data are dynamically altered

Motion tracking (i.e skeletal tracking) data gathered from the driver whilst driving to learn to classify the manoeuvre being	performed (Kinnect)

Support Vector Machine (SVM)

Support Vector Machine (SVM)

main conclusions are: (1) The responsibility approach to distributed coordination is a suitable abstraction for formal reasoning on system safety. The core of this approach is that every entity is responsible for making sure that it does not enter an unsafe state with respect to any other entity. This can be contrasted with the other approaches where consensus is required between all nodes, decisions are made by a central manager, or where each pair of nodes negotiates independently, all of which seem problematic from a scalability point of view; (2) The automatic verification of collaborative vehicular applications with the help of SMT solvers is at least plausible. Some cases were found where the model could not be verified and increasing the detail and scale of the model would certainly enlarge this problem. However, there are certainly domain-specific approximations that can be made to alleviate some of these problems.

MMS-CH algorithm for calculating the safest path in dynamic environment was presented. It used the construction of convex hulls over the input data to eliminate data points irrelevant for the solution and to use the boundary of the hulls to search for the optimal separation margin between sets of obstacles. The tests showed the algorithm performs well in dynamic scenarios where the input data might be altered by insertion or deletion of data points. The preprocessing phase of the MMS-CH algorithm can recognize whether the change in the data set does or does not require any recalculation of the previous solution and thus prevents unnecessary computations.

Preliminary results show that skeletal tracking data can be used in a driving scenario to classify maneuvers.

[41]

[44]

DDT System Model Component – Machine Perception

Thermal infrared classifiers and visible light classifiers were compared and evaluated. Both approaches perform very

Recognize	leading vehicle in a convoy

Prediction	of

advanced		driver assistance	systems

Object detection using Thermal	infrared classifiers and visible light classifiers

ML Classification techniques

Adaptive Boosting (AdaBoost)

Artificial Neural Network

well. However, the accuracy of the visible light classifiers cannot be reached by thermal infra-red classifiers. But because of the good performance of the thermal infra- red classifier under all weather conditions, the performance of the thermal infra-red classifier is acceptable.

SVM shows best ML classification performance (low errors and correlation coefficient near 1) in prognosis of the

[22]

[55]

(ADAS) remaining useful life (RUL) for the prognosis of ADAS’	safety critical components

Road environmental recognition and various	object detection in real driving conditions

Road	Terrain detection

Road	junction detection

Vehicle	Cyber

Single monocular camera for autonomous vehicle in real driving conditions

Color	feature extraction + Clustering algorithm k- mean + ANN

3D point clouds approach

combined to other techniques

Artificial Neural Network combined to other techniques

Artificial Neural Network combined to other techniques

Artificial Neural Network combined to other techniques

Artificial

ADAS systems under the given experimental assumption and Neural Networks have the worst classification performance. This work just proposes a framework for a new area of research in prognostics for automotive domain.

Pedestrian detection algorithm with GPU were 6 times faster than CPU. Traffic sign and traffc light recognition are 2 to 3 times faster than pedestrian detection. However, when the days are dark or there is backlight, it was hard to separate the objects from background.

Color Feature Extraction was used to classify the Road Terrain with a Neural Network (NN). 7666 images were used for classification and results were promising.

The performance of ANNs, SVMs and AdaBoost were compared for the second step of the method, and of HMMs and CRFs for the last. AdaBoost was considered the best classifier, as it managed to learn the training set without overfitting, generalizing well to the test set. On a frame-by-frame analysis, subsequent use of CRF and HMM do not seem to improve from the results obtained by AdaBoost itself. However, both methods removed a lot of the classification noise, generating an output that allows to more clearly detect the start and end of a road junction.

Paper aimed to address the problem of attack detection and identification when the majority of multiple sensors was attacked in an automotive CPS. LSTM and GRU detected and identified attacks by considering sequential information with real data. It was demonstrated that the accuracy of LSTM is the highest among data-based methods (i.e., Neural Network, SVM,

[27]

[31]

[30]

Attack	Detection System

Image	Recognition

Neural Networks

Artificial

simple RNN, GRU and LSTM). The accuracy of LSTM followed the accuracy of GRU. Especially, LSTM and GRU had superior ability to detect coinstantaneous attacks. LSTM and GRU showed high performance in identification of Class 2, 3, 4, 5 and 6. Although calculation time of GRU is faster than that of LSTM, it is no matter to detect the attacks of the sensors on a general computer to use a CPU.

This paper proposed the flushing light detection for preceding vehicles. The results show that the obtained classifier detects turn

[39]

Turn	Signal Recognition

and Timming (95% and 82.2% accuracy)

Neural Networks

signals with an accuracy of 95(%). Moreover, the proposed method is capable of recognizing an appropriate frequency of flushing light with an accuracy of 82.2(%) for sequential driving data.

[18]

Road	Sign

Classification	in Real-time

Spatio-temporal

Use ANN (2 steps: MLP + SLP)

Artificial Neural Networks

Artificial

A novel approach based on two modules was presented. The first module consists of classifying the road sign's shape using MLP. The shapes are classified in four classes: triangular, inverted triangular, circular and octagonal shapes. The accuracy of the MLP, however, is improved when using only six features values for increasing the speed of the algorithm and minimizing the memory. The second module is reserved to the identification of the contents of recognized shapes: the circular and triangular signs via a simple SLP. As for the octagonal sign and upside down triangular, they have a unique indication which are the obligation to stop and give way. A Performance Factor was introduced in order to make a subjective comparison between our proposed approach and the other methods available in the literature, which revealed that our proposed system outperforms most of the other approaches. Regarding running time, the current software implementation takes relatively a real time.

Given a driving video, the research aimed to model which of the surrounding vehicles are most important to the immediate driving task. Employing human-centric annotations allowed for gaining insights as to how drivers perceive different on-road objects. Although perception of surrounding agents is influenced by previous experience and

[32]

situation awareness	Deep Learning

Pedestrian detection

Neural Networks

Artificial

driving style, we demonstrated a consistent human-centric framework for importance ranking. Extensive experiments showed a wide range of spatio-temporal cues to be essential when modeling object-level importance. Furthermore, the importance annotations proved useful when evaluating vision algorithms designed for on-road applications and autonomous driving.

Paper presented a method to differentiate the motion of pedestrians in real life environments. Using a novel input-filtered image based on the post-processing of static recorded video frames, it could successfully distinguish three different pedestrian movement directions. Additionally, it has been proved how CNNs can impressively perform in such a task by training them with

[33]

and	movement direction recognition

Deep Learning

Neural Networks

a specialized dataset. Moreover, it has been demonstrated how the results can be enhanced even further by searching for the best hyper-parameters once the CNN has been fine-tuned for the specific problem, in this case tuning the momentum and weight decay CNN parameters. Paper also presented an evaluation of the current state- of-the-art CNNs, with ResNet being the best-performing CNN for the image

[26]

Pedestrian Trajectory Prediction

Early detection of faults or malfunction

Securing connected vehicles against Denial of Service (DoS) attack

Reliable and robust obstacles detection

Self-learning Trajectory Prediction

On-chip	sensor diagnosis

Augment       message

authentication with Particle Filter (PF) and it to Kalman Filter (KF).

An innovative multi- dimensional structure based on association

Artificial Neural Networks

Artificial Neural Networks

Estimation Filters (e.g. Kalman Filter and Particle Filters)

Estimation Filters (e.g. Kalman Filter

recognition problem used (94% accuracy in the validation set and 79% in the test set).

Results show that the LSTM prediction model is superior to a constant velocity Kalman Filter for pedestrian prediction even on small datasets. Also, it was showed that the prediction model can adapt to changes in the pedestrian walking path using only a small part of the new data. By that, the size of the dataset can be kept rather small although depicting the pedestrian’s movement patterns

The paper discusses the suitability and feasibility of enhancing the reliability of microsensor by adding an on chip self- diagnosis capability. The approach used AI techniques and sensors with no accessible internal signals are taken as an example. Some common acceleration sensor faults are considered, and an indication is given of the manner in which these faults can be detected and isolated, either on an individual sensor basis or based on cooperative work within a sensor network. The design requirements for such self- diagnosable measument systems are set and further practical implementation issues are raised.

Particle filter significantly reduces communication overhead while keeping the same detection level of spoofed messages when compared to Kalman filter in VANET applications. Stimulating different scenarios with Context adaptive beacon verification along with Kalman and particle filter on University of Massachusetts Dartmouth and State Road (Dartmouth) proved that it can detect and prevent spoofed attacks and help reducing the computational overhead. But, the Current method of securing the connected vehicle with filters leave the burden of privacy protection on VANET. The practice makes the autonomous cars the target of attack because of the number of spoofed messages missed by context adaptive beacon verification is around 11% (41 out if 46 were detected) which leaves the undetected rate too high to be replaced by conventional verification method. KF is good when road was linear and lags when the path is non-linear. PF is good for both. KF saves upto 86.5% while Particle Filter can save 85.94% computational overhead for the same scenario. Detect around 76% (24% missed) spoofed beacons with Kalman Filter and 89% (11% missed) spoofed beacons with Particle Filter.

The presented system was able to track and fuse obstacles coming from a laser and a stereo camera. The approach has been

[28]

[55]

[51]

[25]

costs originating from a classifier provides an optimal solution to the association problem with respect to the total association cost.

and Particle Filters)

compared with other state of the art algorithms, showing better results in all the considered metrics. Moreover, the system uses less computational resources and thus, fixed the platform, may work at higher frame rate compared to other solutions, making it more appropriate for automotive applications. The system has demonstrated a correct reconstruction of the dynamic world surrounding the vehicle, proving to be able to help the driver in the assessment of critical situations. In particular, the developed algorithm provides a stable, robust and reliable detection, classification and tracking of the multiple targets coming from different sources. Moreover, the proposed approaches were seen to outperform the state-of-the-art approaches on	a	public	dataset. A fault tolerant and reliable system requires sensors redundancy and complementarity. Common approaches rely on object level fusion. It has been introduced a medium- level fusion which take advantage from both the approaches. The fusion is performed at object level but preserving the low-level information; in this way it is guaranteed a real-time processing exploiting all available information.

To address security issues of a system of connected vehicles (CVs), a fuzzy detector was also introduced that detects possibility of a cyber threat and takes proper actions in

Cyber Attack in V2X	Fuzzy Detector	Fuzzy Logic

response to the specific attack. Results show the designed system can detect any adversary access to the system and can prevent subsequent crashes by adjusting the safe following distance.

A high-level data-fusion strategy has been devised, which is based on the identification and representation of the descriptive and procedural knowledge required. Such a strategy yields better recognition results by merging the various hypotheses generated by the separate

[54]

Road and Obstacle Detection

Sensor Fusion	Fuzzy Logic

Develop		generic technology	that	will

channels and solving possible conflicts through a fuzzy representation of knowledge when compared to a benchmark. In addition, the data-fusion system has performed a more accurate segmentation process by using goal-driven low-level procedures, according to which the image regions have been assigned relative fuzzy memberships to the object to be recognized.

This paper provided an overview of the

[24]

Enhance	Image Understanding

enable the construction of complete, robust, high performance image understanding systems

General AI/ML

technical and program management plans being used in evolving the proposed technology, but no results were presented.

[19]

Learn	from Demonstration

Vehicle	Detection and Counting

Safety-warning and driver-assistance system		and		an automatic pilot for rural	and	urban traffic environments

Speed	bump detection

Obstacle clusteering and tracking

Predicts the CPU usage patterns of software	tasks running on a self- driving car

to support a wide range of DoD applications

Use a Semi-automated mine robot

Hidden	Markov Model + Viterbi algorithm + Adaboost trained Haar-like feature detector Approach

Adaptive randomized HT (RHT) for robust and accurate detection of lane markings without manual initialization or a priori information under different	road environments

Use Camera and image recognition

Lidar + split-and- merge algorithm

Methods for learning the patterns of tasks’ CPU utilizations in given driving contexts

Hidden Markov Based Models (e.g. Continuous Hidden Markov

Model-CHMM and Discrete Hidden Markov

Model- DHMM)

combined to other techniques Hidden Markov Based Models (e.g.

Continuous Hidden Markov

Model-CHMM and Discrete Hidden Markov

Model- DHMM)

combined to other techniques

Hough Transformation

Novel Image Recognition Technique

Ramer- Douglas- Peucker or Ramer- Douglas algorithm

Regression Based Models

In this paper, three methods were compared based on three trajectories in the low noise and noisy environments. The GMM based method had the best performance in a low noise environment. In practice, there’s always unexpected noise around a robot, implying the GMM based method was not practical for real environments. The CHMM based method was suitable for turning trajectories, while The DHMM based method was more robust for straight trajectories.

The proposed method has been shown to give significantly better vehicle volume counts than both multiple targets moving object tracking and VDL on a dataset of over 88 hours of video. On this testing set, the proposed method achieved a median 5- minute-bin error of 0.0686 for this counting task while the multiple target motion tracking and VDL implementations had median errors of 0.0957 and 0.2290 respectively. The proposed method was also more reliable having fewer and less severe occurrences of 5-minute-bin errors throughout the testing set.

In this paper, a prototype was demonstrated, and tasks of lane detection detailed. Preliminary experimental results in different road scene and a comparison with other methods have proven the validity of the proposed method

The average performance of the system considering only speed bump with proper marking is 85%.

Paper presents a robust platform for implementing a perception system for ground vehicles using a LIDAR sensor and two cameras has been designed. Tests were performed using this platform and different implementations, and the results were checked with the real-world scenes, demonstrating the technique validity.

A feature vector was designed to represent the internal and external states of a self-driving car and five regression methods were used to predict the CPU usage patterns of four software tasks running on a self-driving car. Through testing with the actual driving data, the results showed a regression method could be used to predict a software task’s dynamic

[45]

[36]

[21]

[34]

[23]

[53]

Providing road safety to connected drivers and	connected autonomous vehicles

Road Detection

Traffic	Light Detection

Pedestrian Detection

Observing	the Doppler profile

Road area detection method using a support vector machine (SVM) and histogram of oriented gradient (HOG) features and 3D lidar

Two-stage preprocessing		using Principal	Component Analysis		(PCA) followed by SVM

Use High-Definition 3D Range Data (from a LIDAR)

Support Vector Machine (SVM)

Support Vector Machine (SVM)

combined to other techniques

Support Vector Machine (SVM)

combined to other techniques

Support Vector Machine (SVM)

combined to other techniques

CPU utilization.

The paper presented a collision and driving scenario classification system based on the Doppler profile which could potentially decouple the safety benefits of V2V communications from relying on SM content. The Doppler profile in V2V networks showed rich data about the vehicles and their environments and could be exploited to potentially provide a reliable collision avoidance service directly from the radio front-end. No experimental results were presented.

Classifier to differentiate road areas from other areas using a 3D Lidar with machine larning techniques. Range data of lidar changes in the layer direction but not in the rotational direction. HOG features of the reaos concentrate in the same bin. In contract the features of the non-road areas are distributed among several bins representing various directions. Found differences between the histograms for the roas planes and the other areas. In real world data, same tendences of HO features. Error rate of 8.51% using SVM. Area up to 10m ahead of the vehicle can be identified correctly.

Paper presents a system that can detect multiple types of green and red traffic lights accurately and reliably. Color extraction and blob detection were applied to locate the candidates with proper optimization. A classification and validation method using PCANet was then used for frame-by-frame detection. Multiobject tracking method and forecasting technique were succesfully employed to improve accuracy and produced stable results.

An exhaustive analysis of the performance of three different machine learning algorithms have been carried out: k-Nearest Neighbours (kNN), Naïve Bayes classifier (NBC), and Support Vector Machine (SVM). Each algorithm was trained with a training set comprising tool

277 pedestrians and 1654 no pedestrian samples and different kernel functions: kNN with Euclidean and Mahalanobis distances, NBC with Gauss and KSF functions and SVM with linear and quadratic functions. LOOCV and ROC analysis were used to detect the best algorithm to be used for pedestrian detection. The proposed algorithm has been tested in real traffic scenarios with 16 samples of pedestrians and 469 samples of non-pedestrians. The results obtained were used to validate theoretical results. An overfitting   problem   in   the   SVM   with

[20]

[75]

[35]

[27]

How to "automate" manual annotation for images to train visual perception for AVs

Training		visual models using photo- realistic	computer graphics

Support Vector Machine (SVM)

combined to other techniques

quadratic kernel was found. Finally, SVM with linear function was selected since it offered the best results. A comparison of the proposed method with five other works that also use High Definition 3D LIDAR to carry-out the pedestrian detection, comparing the AUC and Fscore metrics. Conclusions are the proposed method obtains better performance results in every case. Pedestrian detection has traditionally been performed using machine vision and cameras, but these techniques are affected by changing lighting conditions. 3D LIDAR technology provides more accurate data (more than 1 million points per revolution), which can be successfully used to detect pedestrians in any kind of lighting conditions

The experiments showed that virtual- world data is effective for training vision- based pedestrian detectors which can be adapted to operate in real scenarios. The different adaptation procedures have shown to provide adapted detectors that improve those trained only on virtual data, as well as those trained using only the real-world data available for the adaptation (which constrained to save a ∼90% annotation effort along the experiments).

[37]