# Application of IoT and Artificial Intelligence in Road Safety

1st Srimantini Bhattacharya

Dept. of Civil Engg NIT Durgapur Durgapur,India

srimantini2019@gmail.com

2nd Harsh Jha

Dept. of Civil Engineering NIT,Durgapur Durgapur, India

harsh002jha@gmail.com

3rd Radhikesh P. Nanda Dept. of Civil Engg. NIT Durgapur Durgapur, India

ORCID-0000-0002-4005-6821

Abstract—This paper explores the advancement of the Internet of Things (IoT) and Machine Learning in the field of Road Safety and accident prevention with a state-of-the-art review of various techniques adopted for implementing an intelligent road Safety System. In this review, emphasis is given on the behavior of drivers, the condition of vehicles (two- wheelers, four-wheelers), health condition of roads and bridges, and theft-related issues using Radio Frequency Identification (RFID). It is seen that, with the help of IoT, the safety system can be updated on a real-time basis which can help to create a smart, intelligent, and highly efficient Road Safety system. Artificial Intelligence (AI) is applied to enhance the technology further for detecting the driver's behavior like drowsiness with the help of real-time camera feed or high- resolution images. Additionally, the role of AI in detecting the condition of roads and bridges in preventing road accidents is also discussed. Though the paper provides a good insight into the application of IoT and Machine Learning in the smart Road safety system, certain limitations are highlighted.

Keywords— Internet of Things (IoT), Artificial Intelligence (AI), Artificial Neural Network (ANN), Rasberry Pi (RPI), Radio Frequency identification (RFID), Global Positioning System (GPS), Intelligent Transportation System (ITS).

- INTRODUCTION

With the exponential growth of urbanization in India, there is also an increase in traffic congestion in day-to-day life, and the traditional traffic management system is ineffective [1,2]. This makes road safety one of the prime importance because of the rise in deaths due to crashes or accidents on the road. With the number of vehicles on our roads increasing with every passing hour, it's vital to come up with an optimal solution that can be effective in maintaining the safety of the people on the road and thereby reducing the chances of deaths in the country. With the expeditious growth of technology, various methods of Road Safety have been developed all across the world using the Internet of Things and Machine Learning to make the analysis systematic and efficient [3]. Researchers as stated in [4,5] have extensively reviewed two main fields to prevent road crashes–(a) The behavior of driver under various conditions,

(b) pieces of information like driving speed, frequency of application of brakes, diverse engine and transmission system parameters under different peripheral conditions.

The majority of the methods developed involves three segments - Use of Machine Learning Model for analyzing the behavior of the drivers (First Segment), Collection of data from multiple sensors and connecting them to various

servers, network devices for processing and transmission of data using Internet of Things (IoT) (Second Segment), Development of a user-friendly interface such as Android App for the users (Third Segment). IoT and Machine Learning are some of the booming technologies of today, helping in intelligent monitoring, reporting, and control of every aspect of our lives.

The traffic safety system of a metropolitan city is a major parameter for urban mobility. This paper presents a review of application of IoT and Machine Learning in road safety and smart road system. Next part of the paper focuses role of IoT and Artificial Intelligence in detecting drowsiness, consumption of alcohol, over-speeding or missing safety gear such as seat belts, helmets by the driver thereby avoiding deaths or causalities. Finally, the limitations of such technology in road safety are described. This research produces a system proposal that can be used in further research and can be applied in terms of the development of smart cities, especially smart transportation.

- LITERATURE SURVEY

With the rapid growth of modernization, there is a havoc rise of traffic, which increases congestion and leads to severe crashes and accidents, putting many lives in danger. Approximately 1.35 million people lose their lives every year due to accidents, and around 50 million suffer from severe injuries. With India focussing on building smart cities, this is a major concern for India to find an effective solution. The best way to implement IoT and Machine Learning techniques and make a smart Road safety system that will analyze the physical parameters of vehicles and study the drivers' behavior that will efficiently reduce congestion, reduce the time of travel, and help prevent crashes.

IoT and Machine Learning are both the most booming technologies of today. IoT describes communication between various physical objects, embedded with sensors, software for analyzing and transmitting information between systems with the help of the internet. IoT makes it possible for users to collect and analyze data with minimum human intervention making the process cost-effective due to easily available sensors and efficiency. Devices and sensors are connected to an IoT Platform that integrates data and applies analytics to give the most valuable information. These IoT platforms make it easy to transmit useful pieces of information that can be used for detection and make recommendations for a particular problem. Another growing

2022 Interdisciplinary Research in Technology and Management (IRTM) | 978-1-6654-7886-1/22/$31.00 ©2022 IEEE | DOI: 10.1109/IRTM54583.2022.9791529

978-1-6654-7886-1/22/$31.00 ©2022 IEEE

technology is Machine Learning, a branch of artificial and computer science that focuses on using data and algorithms the way humans learn with accuracy. Machine Learning uses data, various algorithms, and statistical methods to understand, recommend, classify and detect different situations with high accuracy. Thus, we can use these booming technologies to make a highly efficient and user- friendly road safety system.

For example, as discussed in [6], to tackle the problem of traffic congestion leading to crashes and accidents due to the increase in population during the 2022 World Cup in Qatar, the naturalistic driver behavior is utilized to collect and analyze the data for traffic planning to maintain safety. An IoT-based solution is implemented, which collects vehicle data like - time of the trip, GPS location, maximum, minimum, and average speed to study the data and predict accidents and road infrastructure development to prevent crashes. The driver's behavior is also considered, including the driver's drowsiness level. The figure 1 from [7] describes the alarming situation in Road Safety due to Driver drowsiness. A framework and a deep learning-based application are implemented to check the drowsiness level of the driver to an accuracy of 82%.

Fig-1 Accidents due to drowsiness

In [8], a cost-effective, novel IoT architecture is made with robust computational methods in assessing road safety. This safety system approach to road safety is now adopted worldwide. However, the considerations are made for the medium-to-long term. This work implements the approach for a short-to-medium-term dynamic assessment of road safety. In addition, the use of machine learning in the design of the computational core is show-cased by using an application of Hidden Markov Models. The proposed architecture is demonstrated through an application to safety-based route planning.

Machine Learning also plays a vital role in predicting accidents. Prediction of the automotive accident severity is important for an intelligent traffic system. In [9] some classification models like Logistic Regression, Artificial Neural network, Decision Tree, K-Nearest Neighbors, and Random Forest have been implemented to predict accident severity. A secure communication architecture model has been explained to exchange information among the devices, and a web-based alarm system is introduced that can alert the user for any accident. However, these techniques require a wide range of land data as input, reducing to a low

resolution and losing detail. So, to solve this problem, a solution as stated in [10] has been proposed to use a high- resolution data framework and the application of IoT is the deep learning architecture of Convolution Neural Network (CNN). It is found that high resolution of data provides a result with more accuracy.

Several solutions have been presented to solve the problem of tackling traffic congestion [11]. One of these methods is to solve this problem through Artificial Neural Networks (ANN). Modeling Smart Road Traffic Congestion Control using	Artificial	Back	Propagation		Neural	Networks (MSR2C-ABPNN) is proposed [12] with the help of ANN for road traffic regulation and control. The proposed MSR2C-ABPNN based model predicts the point of congestion using neural network and backpropagation algorithm. It is further divided into two sub-layers The first layer, i.e., prediction layer, backpropagation, is used to detect the occupancy. In contrast, the second layer, i.e., the performance	layer,	evaluates	the			prediction	layer performance in terms of RMSE, Accuracy, and Miss Rate. Tab-1 from [13] shows the statistics of law-breakers in two- wheelers and four-wheelers which is a major reason behind accidents. In [14], smart accident prevention and detection is proposed		using		V2V		communication	technologies, Raspberry Pi, various sensors, MEC architecture, etc. for the prevention of two-wheeler accidents.		This system detects accidents	by		vibration		sensors,		accelerometers.		For detection, the GPS and GSM modules locate the accident site and correspondingly inform the person’s nearest ones and nearby hospitals through a text message. The system also requires the person that will be riding the bike to have a valid driving license using the already embedded RFID on the driving license. The RFID reader on the bike will have at most ten registered users, hence handling theft-related issues. The system is efficient in terms of both the parameters and performance.

Table-1 Statistics of Law Breakers on two and four-wheelers

| Law breakers | Two-wheelers | Four-wheelers |
| --- | --- | --- |
| Signal jump | 2,20,859 | 1,46,945 |
| Drunken drive | 36,727 | 17,237 |

Furthermore, to prevent accidents, an alerting system has been set up as proposed in the [15]. The prototype is designed using Raspberry Pi, Pi Camera, sensors for monitoring driver's eye movements, detecting yawning, detecting toxic gases, and alcohol consumption to prevent accidents and to provide safety assistance to the drivers. Thereafter the Internet of Things (IoT) and Machine Learning (ML) enabled system is implemented in vehicles for transmitting the behavior of the driver and his driving pattern to the cloud to take quick response under emergencies. Cloud services and machine learning are employed in identifying the fatigue of the drivers through the collected dataset. Various Machine Learning and IoT based algorithms work simultaneously to smoothly execute all the different prevention methods. The general architecture model is also shown below in the fig 2[13]. The device is experimentally tested, and the results show its efficiency and effectiveness.

Fig-2 General architecture of the model

A model was proposed [16] to incorporate ITS context incorporating speed and pollution adaptive traffic control systems with weather advisories. The Scalable Enhanced Road Side Unit (SERSU) was developed for use as part of a comprehensive Intelligent Transportation System based on the Internet of Things (IoT) concept. It collects information using a variety of sensors and an onboard camera. The collected data can then be uploaded to a central server for speed limit adjustment, metering routes to reduce vehicle congestion and emissions, and issuing weather advisory warnings. Wireless cellular networks and RF modules are used for communication with all the devices.

To prevent accidents on the road, Smart IoT cars are proposed in [17] in which we can control distinctive things or keep track of a vehicle for security, solace, and proficiency. Technologies such as ease liquor sensor, safety belt consisting of an inbuilt heartbeat sensor, and edge restrict are designed to prevent mishaps from occurring. All controls are accessible in auto proprietors' dashboard accessible both in auto and in a versatile application.

Another prominent factor that contributes to fatal accidents is the behavior of drivers. Often due to driver drowsiness or drunken driving, major accidents are caused, which contributes to a large number of deaths. So it is of grave importance to solve this problem. With the help of the Q- 135 sensor [18], alcohol can be detected from the air, so whether the driver is drunk or not can be checked. The sensor sends the signal to ARM-7. If the driver is drunk, then vehicle ignition will not start until the driver is not changed. In case the car is already in driving condition, then the system alerts the driver using a buzzer. Another human behavior that contributes to road accidents is microsleep. This is due to the current trend of life like heavy workloads, long working hours, excess consumption of caffeine, and many others causing massive fatal accidents. The creation of SMART Vehicles in the IoT increases the technology

capabilities in reducing the number of crashes on the roads. An integration with Artificial Intelligent (AI) [19] can be used to detect microsleep. While the image processing can see the face changes from normal to microsleep symptoms by tracking the eye degree, the head motion, and the mouth yawning.

Another critical aspect leading to road accidents is low visibility [20]. It can also be caused by other accidents, work-in-progress on roads, excessive motorized vehicles, especially at peak times, and so on. Fixed traffic sensors are installed on roads that interact with drivers' mobile apps through the 4G network to solve this problem. This, however is not feasible on all roads. To make it work for non-highway roads, mobile traffic sensors can be installed in private and public transportation and volunteer vehicles. An IoT Cloud system based on OpenGTS and MongoDB is proposed to solve this problem and assist critical rescue vehicles such as ambulances, etc. It is also shown below in fig 3. Experiments run on this application show-case that this system shows the acceptable response time to be sent to drivers to mitigate this problem by avoiding accidents.

Fig-3 IoT Cloud system for traffic monitoring and alert notification.

- SIGNIFICANCE OF SMART ROAD SAFETY SYSTEM

As per the United Nations 2030 plan, there is a requirement for an enhanced transportation system with highly effective road safety. This can be accomplished with the assistance of IoT and Artificial Intelligence. The primary objective of IoT and Artificial Intelligence to be used in Road Safety is to connect the traffic system using real-time data, which helps all the traffic information be updated efficiently and simultaneously. IoT can play a significant role in collecting all the necessary data from the vehicle, such as trip details, speed limit, GPS, etc. This can help prevent crashes and enhance safety very efficiently. Similarly, with the help of Artificial Intelligence, we can track the driver's behavior and the pedestrian that can help us prevent accidents. These methods can be highly efficient for maintaining an advanced traffic system with effective road safety. In [21], a discussion is made on the effective implementation of these budding technologies to provide a reliable, intelligent, and smart experience to the users traveling along the highways. The study is divided into five categories- smart highway lighting system, smart traffic and emergency management system, renewable energy sources on highways, smart

display, and Artificial Intelligence in Highways. This study has also discussed the enhancement of Road safety and sustainable experience for travellers by integrating deep learning techniques in the edge-based vision node for studying the pattern of traffic flow, road condition, and road safety.

Apart from these Internet of Things and Artificial Intelligence can also play an important role in detecting drowsiness, the behavior of drivers after consumption of alcohol, over speeding or missing safety gear such as seat belts, helmets can help us to avoid crashes and thereby avoid any serious deaths or injuries. Below are some reasons that cause crashes that have been discussed which can be optimised with the help of IoT and Machine Learning.

Over Speeding:

This is one of the common reasons behind crashes. Humans tend to Overspeed and often lose control, leading to fatal accidents. So to tackle this problem, a model can be used as stated in [22]. A micro-controller and a radio frequency identification system are used to limit the overspeeding of vehicles. Automatic speed reduction can be achieved with the help of the transmitter and receiver modules when the vehicles are in an accident-prone region.

Driver's behavior:

Another important reason behind fatal road accidents is the distraction of drivers or drowsiness. This problem can be tackled by a novel ensemble approach based on deep learning, as stated in [23], which can detect a distracted driver by detecting the objects involved in distracting the driver using Faster-RCNN. This has achieved a validation accuracy of 97.7%. Moreover, to reduce any false alarms, the pose points of the driver are also taken into consideration to make sure only those objects are detected which are directly associated with the driver's distraction. The proposed method is not only time-efficient, robust but cost-efficient as well. Apart from this, with the help of Deep Learning, we can also detect the drowsiness of driver as stated in [24] with the help of an architecture consisting of 4 deep learning layers - AlexNet, VGG-FaceNet, FlowImageNet, and ResNet, which uses videos of drivers as input and detect drowsiness. Using a Soft-Max classifier, it also considers hand gestures, facial expressions, behavioral features, and head movements to detect drowsiness with almost 85% accuracy.

Lack of Safety Measures:

With the increase in the number of vehicles on the road and traffic congestion, the absence of safety gears like helmets and seat belts have become a prominent reason for an accident. The best way we can tackle this is with the help of IoT and machine learning, like a smart helmet system [25] in which IR and alcohol detection sensors are used to detect the presence or absence of a helmet. Similarly, a seat belt detection system [26], with the help of the YOLOv3 target detection algorithm and the lightweight network structure, can be used to prevent road accidents and thus decrease deaths.

- ADVANTAGES OF IOT AND ARTIFICIAL INTELLIGENCE IN

ROAD SAFETY

The transportation authority is associated with a high maintenance cost, fatal accidents, injuries, and loss of life every year. For a developing nation like India, it is vital to find a solution. The best way to tackle this problem is with the help of the booming technologies of today like Machine Learning and artificial intelligence. These technologies can help optimize the maintenance cost and prevent fatal accidents, thus saving the lives of civilians.

Detecting human behavior:

One of the major reasons that contribute to fatal accidents is human behavior and their negligence to follow the traffic rules. IoT can play a proactive role in helping drivers adopt safety rules. With the help of IoT, the traffic management system gets updated with real-time data, thus increasing the efficiency of the safety system. With the help of Machine Learning and IoT, we can judge the driver's behavior and alert them simultaneously. For example, in [27], a deep learning model is proposed to explore the complex interactions between the driver's behavior and the road environment. The proposed model consists of an unsupervised Denoising Stacked Autoencoder (SDAE) that will provide output layers in RGB colors. The SDAE model is also shown below in the figure. With the help of the graphical outcome, the patterns of simple driving behavior and road environment complexity can be detected with better efficiency.

Detecting Condition of Roads and bridges

Another factor that influences road accidents is the poor condition of roads. With the help of IoT, we can continuously monitor the condition of roads so that maintenance can be done accordingly, which will help us prevent crashes. With the help of Machine Learning, we can identify cracks in bridges and that can help prevent any traumatic disasters. One such system can be an Intelligent speed breaker system as proposed in [28], which helps detect speed breakers that might get neglected by drivers due to bad visibility or over-speeding. In this system, an RF module is used that warns the driver about the existence of a speed breaker in proximity and also assists in automatically reducing the vehicle’s speed if no action is taken in time. Through the IoT, GPS location of speed breaker can be sent to the cloud using GPS and stored on the cloud to use it for future. Additionally, with the help of ML, we can also perform pothole detection as shown in [29], in which accelerator and gyroscope, which are usually available in all smartphones, are used to collect road status information and use Transfer learning for detecting the potholes.

Shortly, IoT and Machine Learning can greatly assist people in preventing road accidents. These technologies will help develop a smart, efficient, and intelligent traffic system that will help the countries reduce their mortality rate due to road accidents and help the transportation authority reduce their maintenance cost and help them prevent disaster. Thus these technologies prove to be a boon in the field of road safety.

- LIMITATIONS OF IOT AND ARTIFICIAL INTELLIGENCE IN

ROAD SAFETY

Although the concepts of IoT applying to road safety sound exciting and very much increase road safety, these applications are still not in use. This can be accredited for several reasons. The most important is the effectiveness [30] of the application not being considered. Most of the studies evaluating IoT technologies to prevent accidents on construction sites have not investigated their effects, which presumably prevents construction companies from adopting IoT technologies. The current body of knowledge lacks a method for quantitatively evaluating the effectiveness of such technologies, which probably is one reason why they have not been deployed widely. Secondly [31], infrastructural, issues such as road zoning, planning, & other construction-related concerns become significant problems for implementing this technology. Thirdly, high-speed Internet-oriented data transfer media is required to apply the IoT-based technology. In a country like India, the network unavailability or instability for any reason may disrupt the entire traffic control system. Therefore, overall data management would be a big challenge for this traffic management system. Lastly, many devices are accessing the central network, which increases the chance of hacking or malfunctioning the system. Hence, a top-notch security layer is necessary for making an impenetrable and hack-safe smart traffic application. In addition to this, [32] there is a loss of data when huge unconnected vehicles come between the connected vehicles and at building intersections. Because of more moving vehicles, Doppler effects, shadows, and multiple paths fading occur.

- CONCLUSION

The present study focuses on all the various researches made in road safety with the help of IoT and Machine Learning. It is seen that, with the help of IoT, the safety system can be updated on a real-time basis which can help to create a smart, intelligent, and highly efficient road safety system. AI is applied to enhance the technology further for detecting the driver's behavior like drowsiness with the help of real-time camera feed or high-resolution images. The true power of IoT in ensuring safe driving continues to be unleashed as cars move towards becoming fully autonomous and start interacting with their environment and making decisions on their own. This will prevent drivers from entering hazardous areas, assisting in avoiding collisions, selecting detours, and avoiding traffic congestions. Though research on IoT and ML has not yielded much development in Civil Engineering sector, present study definitely gives a major breakthrough in transportation Engineering particularly for road safety measures.

- REFERENCES

- Javaid, S., Sufian, A., Pervaiz, S., & Tanveer, M. (2018). Smart traffic management system using Internet of Things. 20th international conference on advanced communication technology (ICACT), IEEE, 393-398.

- Rabby, M. K. M., Islam, M. M., & Imon, S. M. (2019), A review of IoT application in a smart traffic management system. 5th International Conference on Advances in Electrical Engineering (ICAEE), IEEE. 280- 285.

- Torbaghan, M. E., Sasidharan, M., Reardon, L., & Muchanga- Hvelplund, L. C. (2022). Understanding the potential of emerging digital technologies for improving road safety. Accident Analysis   & Prevention, 166, 106543.

- Ghandour, A. J., Hammoud, H., & Al-Hajj, S. (2020). Analyzing factors associated    with    fatal    road    crashes:    a    machine    learning approach. International journal of environmental research and public health, 17(11), 4111.

- Vasavi, S. (2018). Extracting hidden patterns within road accident data using machine learning techniques. In Information and Communication Technology, Springer, Singapore. 13-22.

- Jabbar, R., Shinoy, M., Kharbeche, M., Al-Khalifa, K., Krichen, M., & Barkaoui, K. (2019, December). Urban traffic monitoring and modeling system: An iot solution for enhancing road safety. In 2019 International Conference on Internet of Things, Embedded Systems and Communications (IINTEC), IEEE. 13-18.

- https://trafficsafety.ny.gov/drowsy-driving-statistics

- Taha, A. E. M. (2018). An IoT architecture for assessing road safety in smart cities. Wireless Communications and Mobile Computing, 2018.

- Mohanta, B. K., Jena, D., Mohapatra, N., Ramasubbareddy, S., & Rawal, B. S. (2021). Machine learning based accident prediction in secure iot enable transportation system. Journal of Intelligent & Fuzzy Systems, (Preprint), 1-13.

- Cai, Q., Abdel-Aty, M., Sun, Y., Lee, J., & Yuan, J. (2019). Applying a deep learning approach for transportation safety planning by using high- resolution transportation and land use data. Transportation research part A: policy and practice, 127, 71-85.

- Thakur, A., Malekian, R., & Bogatinoska, D. C. (2017, September). Internet of things-based solutions for road safety and traffic management in intelligent transportation systems. In International conference on ICT innovations (pp. 47-56). Springer, Cham.

- Ata, A., Khan, M. A., Abbas, S., Ahmad, G., & Fatima, A. (2019). Modelling smart road traffic congestion control system using machine learning techniques. Neural Network World, 29(2), 99-110.

- Agarwal, N., Singh, A. K., Singh, P. P., & Sahani, R. (2015). Smart helmet. International Research Journal   of   Engineering   and Technology, 2(2), 3.

- Nanda, S., Joshi, H., & Khairnar, S. (2018, August). An IOT based smart system for accident prevention and detection. In 2018 Fourth International Conference on Computing Communication Control and Automation (ICCUBEA) IEEE. 1-6.

- Uma, S., & Eswari, R. (2021). Accident prevention and safety assistance using IOT and machine learning. Journal of Reliable Intelligent Environments, 1-25.

- Al-Dweik, A., Muresan, R., Mayhew, M., & Lieberman, M. (2017, April). IoT-based multifunctional scalable real-time enhanced road side unit for intelligent transportation systems. In 2017 IEEE 30th Canadian conference on electrical and computer engineering (CCECE) IEEE. 1-6.

- Vimalkumar, S., Hemalatha, P., & Kalaivani, J. (2018). A review on smart IOT car for accident prevention. Asian J Appl Sci Technol, 2(1), 287- 292.

- Vyas Viral, M., Choksi, V., & Potdar, M. B. (2017). Car Safety System Enhancements using the Internet of Things (IoT). International Research Journal of Engineering and Technology (IRJET) 04 (12).

- Zaleha, S. H., Wahab, N. H. A., Ithnin, N., Ahmad, J., Zakaria, N. H., Okereke, C., & Huda, A. N. (2021, December). Microsleep Accident Prevention for SMART Vehicle via Image Processing Integrated with Artificial Intelligent. Int. Journal of Physics: Conference Series, 2129(1),.

IOP Publishing. 012082

- Celesti, A., Galletta, A., Carnevale, L., Fazio, M., Ĺay-Ekuakille, A., & Villari, M. (2017). An IoT cloud system for traffic monitoring and vehicular accidents prevention based on mobile   sensor   data processing. IEEE Sensors Journal, 18(12), 4795-4802.

- Singh, R., Sharma, R., Akram, S. V., Gehlot, A., Buddhi, D., Malik, P. K., & Arya, R. (2021). Highway 4.0: Digitalization of highways for vulnerable road safety development with intelligent IoT sensors and machine learning. Safety science, 143, 105407.

- Sharma, S., Kishore, I. S., Mishra, S., Girotra, D., & Talwar, H. (2020). Automatic vehicle speed reduction & accident prevention system using RF technology. Int. Journal of Engg. Applied Sc. and Technology, 5(7), 292-295.

- Draz, H. U., Khan, M. Z., Khan, M. U. G., Rehman, A., & Abunadi, I. (2021, April). A Novel Ensemble Learning Approach of Deep Learning Techniques to Monitor Distracted Driver Behaviour in Real Time. In 2021 1st International Conference on Artificial Intelligence and Data Analytics (CAIDA), IEEE. 251-256.

- Dua, M., Singla, R., Raj, S., & Jangra, A. (2021). Deep CNN models- based ensemble approach to driver drowsiness detection. Neural Computing and Applications, 33(8), 3155-3168.

- Rahman, M. A., Ahsanuzzaman, S. M., Rahman, I., Ahmed, T., & Ahsan, A. (2020, June). IoT Based Smart Helmet and Accident Identification System. In 2020 IEEE Region 10 Symposium (TENSYMP), IEEE. 14-17.

- Wang, Z., & Ma, Y. (2021). Detection and recognition of stationary vehicles and seat belts in intelligent Internet of Things traffic management system. Neural Computing and Applications, 1-10.

- Bichicchi, A., Belaroussi, R., Simone, A., Vignali, V., Lantieri, C., & Li, X. (2020). Analysis of road-user interaction by extraction of driver behavior features using deep learning. IEEE Access, 8, 19638-19645.

- Biswal, S., Chandra, I., Sinha, S. K., & Pandey, K. (2021). Intelligent speed   breaker   system   design   for   vehicles   using   Internet    of Things. Materials Today: Proceedings.

- Song, H., Baek, K., & Byun, Y. (2018). Pothole detection using machine learning. Advanced Science and Technology, 151-155.

- Yeo, C. J., Yu, J. H., & Kang, Y. (2020). Quantifying the effectiveness of IoT technologies for accident prevention. Journal of Management in Engineering, 36(5), 04020054.

- Rabby, M. K. M., Islam, M. M., & Imon, S. M. (2019), A review of IoT application in a smart traffic management system. 5th International Conference on Advances in Electrical Engineering (ICAEE) IEEE. 280- 285.

- Devi, Y. U., & Rukmini, M. S. S. (2016). IoT in connected vehicles: challenges and issues—A review. International Conference on Signal