IET Intelligent Transport Systems

Research Article

# Real-time crash prediction on urban expressways: identification of key variables and a hybrid support vector machine model

Jie Sun1, 2, Jian Sun1, 2 ✉

ISSN 1751-956X

Received on 26th November 2015 Revised on 28th December 2015 Accepted on 17th January 2016 doi: 10.1049/iet-its.2014.0288 www.ietdl.org

1Key Laboratory of Road and Traffic Engineering, Ministry of Education, Tongji University, No. 4800, Cao’an Road, Shanghai 201804, People’s Republic of China

2Jiangsu Province Collaborative Innovation Center of Modern Urban Traffic Technologies, SiPaiLou #2, Nanjing 210096, People’s Republic of China

✉ E-mail: sunjian@tongji.edu.cn

Abstract: The traffic safety on expressways is crucial for the efficient operation of the expressway system, and there is a close relationship between traffic states and crashes on expressways, and the occurrence of crashes may be influenced by the interaction of different combinations of traffic states upstream and downstream of the crash location. Based on the crash data and the corresponding traffic flow detector data collected on expressways in Shanghai, this study proposes a hybrid model combining a support vector machine (SVM) model with a k-means clustering algorithm to predict the likelihood of crashes. The random forest (RF) model is employed to select the important and significant variables for model construction from the data of the traffic flow 5–10 min before the crash occurred. Then, the cross-validation and transferability of different models (SVM model without variable selection, SVM model with variable selection, and hybrid SVM model with variable selection) are determined using 577 crashes and 5794 matched non-crash events. The results show that the crash prediction model along with the four most important variables selected using the RF model can obtain a satisfactory prediction performance for crashes. With the combination of the clustering algorithm and SVM model, the accuracy of the crash prediction model can be as high as 78.0%. Moreover, the results of the transferability of the three different models imply that the variable selection and clustering algorithm both have an advantage for crash prediction.

# Introduction

Trafﬁc accidents occurred on urban expressways cause not only personal injury and property damage, but also heavy trafﬁc congestion. Speciﬁcally, during peak hours, trafﬁc jams and delays caused by crashes on expressways lead to a great waste of social costs in spite of the low severity of crashes due to heavy trafﬁc demand. With the development of intelligent transportation systems, especially advanced trafﬁc information systems (ATIS), increased attention has been given to predicting the crash risk on expressways and freeways based on trafﬁc data collected from ATIS. During the past decade, studies concerning crash prediction models have been very popular in freeway safety analyses [1–19]. In practice, with a real-time crash prediction model, several safety countermeasures on expressways can be applied. First, drivers can be notiﬁed through in-car display devices or variable message signs on the road; furthermore, in the case of a likely high-risk crash segment, it is important to dispatch rescue vehicles in advance.

In general, the development of a real-time crash prediction model

is based on crash (non-crash) data and the corresponding trafﬁc data, which are collected from detectors installed upstream and downstream of the crash location. In addition, the trafﬁc state of expressways can be described by trafﬁc parameters, among which the basic trafﬁc ﬂow data (i.e. volume, speed and occupancy) and their combinations are commonly used. Furthermore, several previous studies developed different crash prediction models within different trafﬁc states [5, 15, 17]. However, there is a relationship between trafﬁc states and crash incidents on freeways [20]. The occurrences of crashes may be caused by the interaction of different combinations of upstream and downstream trafﬁc states near the crash location. Thus, the crash prediction model should include trafﬁc states in the model rather than be simply developed separately for different trafﬁc states. Therefore, the objective of this research is to categorise various trafﬁc states with

different trafﬁc parameters using clustering algorithm and then to predict crashes with a support vector machine (SVM) model which together compose a hybrid model.

On the other hand, some previous research paid more attention to the crash prediction modelling approach, while the identiﬁcation of the critical variables that can describe the crash risks was ignored [2, 6, 19]. However, it can lead to high computational complexity and over-ﬁtting of models due to too many variables. Thus, it is important to identify the critical variables from the whole candidate variables before using the crash prediction models. Since the random forest (RF) model is an effective method for calculating the importance of the model variables [21], critical variables are selected by the RF model for the construction of the predictive model in this study.

This remaining of this paper is organised as follows: A brief overview of the previous research is presented in Section 2. Section 3 describes the study area and data. The methodology used in this study is introduced in Section 4. Then, the results and discussions are presented in Section 5. Section 6 concludes the paper.

# Literature review

Generally, the objective of a crash prediction model is to classify the pre-crash conditions from the normal conditions as a categorisation process. Therefore, numerous prediction models based on different modelling methods to analyse and predict the crash likelihood on freeways have been used, including different statistical regression methods [1–7, 10–11, 14, 16, 17] and data mining algorithms [8,

9, 12, 13, 15, 18, 19].

Among the ﬁrst studies, Oh et al. [1] adopted non-parametric Bayesian statistics to evaluate the crash likelihood using real-time trafﬁc data and 52 crash records from I-880 in California. It was demonstrated that the standard deviation of the vehicle speeds in

- min time intervals can be used to distinguish 55.8% of crash-prone trafﬁc conditions from normal trafﬁc conditions. Lee et al. [2] built an estimation model adopting an aggregate log-linear analysis with variables such as speed variation, trafﬁc density, road geometric characteristics and other related factors before accidents. The results showed that the short-term turbulence of the trafﬁc ﬂow has a signiﬁcant impact on the occurrence of trafﬁc accidents. Furthermore, a series of studies have been conducted by Abdel-Aty et al. and Pande et al. regarding the prediction of the real-time crash risk, in which the matched case–control logistic regression [3, 4, 6, 7] and the artiﬁcial neural network models [8, 9] were two commonly employed methods. Nevertheless, the best-performing model of their studies could only predict 73.9% of crashes with a false alarm rate of 28.7% [8].

Recently, Hossain and Muromachi [12, 13] employed Bayesian belief net with data collected from detectors on Tokyo expressways to establish real-time crash prediction models for expressway basic segments and ramp segments. With this prediction model, 66% of the crashes can be classiﬁed with an average false alarm rate of less than 20% on basic segments. Pande et al. [6] developed real-time crash risk assessment models for four freeway corridors based on logistic regression and classiﬁcation trees and tested the transferability (the viability of applying a model built on one freeway on other freeways) of the models. The result showed that the prediction model with a logistic regression and classiﬁcation tree can distinguish, respectively, 61.9 and 61.7% of crashes, while the transferability of the models was lower, at 55.84 and 50.37%, respectively.

In addition, models have been established to predict the crash likelihood for particular scenarios, such as different levels of severity [10, 14] or different types of trafﬁc conditions [5, 15, 17]. It was found that the variables entered in the different state-related models were not the same, which is consistent with the probable mechanisms of crashes under the respective trafﬁc conditions [5].

With respect to variable selection, several modelling methods have been used in different research ﬁelds, such as intrusion detection, and a rough set was adopted to identify the important parameters before the construction of the intrusion detection model [22]. In the area of crash prediction, before the models were established for an estimation of crash risk, the classiﬁcation and regression tree (CART) model [18] and random multinomial logit model [12] were also developed to select the most important and explanatory variables.

From the foregoing, there are different modelling methods for the analysis and prediction of crashes on freeways/expressways. However, the prediction performances of previous study are not signiﬁcantly well. Therefore, considering that the SVM model as a classical classiﬁcation method has already been applied in crash frequency [23] and crash severity [24] analyses, it was used to predict the risk of crash occurrence in real time; the SVM model was also demonstrated as an appropriate method in our previous study [19]. However, several issues arise within our previous SVM model:

  - The relationship of different trafﬁc states and crashes was not clear.

  - A relatively high false alarm rate exists even though good crash prediction accuracy can be obtained.

  - The transferability of the crash prediction model is also essential and needs to be improved upon as well as its original performance.

  - The computational efﬁciency of the SVM model may decrease sharply with large-scale data and too many model variables.

Thus, a clustering model is applied to abstract the trafﬁc parameters into several trafﬁc states. Moreover, it is combined with the SVM model to develop a hybrid model and enhance the computational ability for both crash prediction and false alarm performance. Meanwhile, the RF model is constructed to identify the critical variables for the prediction model for the purposes of avoiding over-ﬁtting and improving the computational efﬁciency.

# Data collection

  - Crash data

The research objective of this study is to investigate the impact of the dynamic trafﬁc ﬂow conditions on crash risk within the basic segments of an urban expressway. Considering the potential effects of road geometry and on/off-ramp arrangement and the purpose of transferability test, seven segments with similar road geometries and ramp types on expressways in Shanghai are selected for the study, including four segments on Yan-an expressway and three segments on North–South expressway. The data collected by Yan-an expressway are used for the training and testing of the model, whereas the data collected on North–South expressway are used for examining the transferability of the model. All of these segments are three-lane expressway segments, and the spacing of the detectors ranges from 300 to 500 m. The names and crash statistics of the segments are presented in Table 1. The crash data are extracted from video surveillance systems and the police report to the trafﬁc management centre, while the trafﬁc ﬂow data are collected by dual-loop detectors installed on expressways. The collected crash and trafﬁc data cover the period from April 2010 to December 2010. A total of 577 crashes in normal weather condition are identiﬁed and used in the study.

Each crash data contain information, such as the accident occurrence time, location, type of crash and weather.

  - Traffic flow data

The trafﬁc state of an expressway in this study is described by the trafﬁc ﬂow parameters (volume, speed and occupancy) and their combinations. These data are collected from the detectors near the crash site.

Before the crash occurrence, there are nearly 20 min of trafﬁc ﬂow data that can be used for crash prediction, which could be separated into four 5-min time slices [6]. The trafﬁc data 5–10 min before the crash are used for modelling in this study as suggested by the studies in [3, 19].

Each crash corresponds to the trafﬁc data collected from the four detectors nearest to the crash, each with two detectors upstream and downstream. The four detectors are named D1, D2, D3 and D4 in the order from upstream to downstream, as shown in Fig. 1. Therefore, trafﬁc ﬂow data (volume, speed and occupancy) can be obtained

Table 1   Summary of the study sites and crash data

| Expressway | Segment on expressway | Length, km | AADT, veh/ day | Number of crashes | % of total crashes |
| --- | --- | --- | --- | --- | --- |
| West bound Yan-an expressway | Yandong Interchange to Maoming | 0.8 | 89,740 | 102 | 17.7 |
|  | Road |  |  |  |  |
| East Bound Yan-an expressway | Hongxu Road to Loushanguan Road | 1.3 | 82,917 | 201 | 34.8 |
|  | Yanxi Interchange to Kaixuan Road | 0.8 | 62,419 | 11 | 1.9 |
|  | Jiangsu Road to Huashan Road | 1.5 | 88,639 | 94 | 16.3 |
| North Bound North–South expressway | Yanchang Road to Guangzhong Road | 1.8 | 60,901 | 32 | 5.5 |
| South Bound North–South | Guangzhong Road to Luochuang Road | 2.0 | 50,154 | 72 | 12.5 |
| expressway | Gongjiang Road to Changzhong Road | 2.2 | 51,751 | 65 | 11.3 |
| Total |  | n/a | n/a | 577 | 100 |

downstream detectors. Speciﬁc variables are listed in Table 2. To aid interpretation, the variables that are directly yielded by the detectors are coded as ‘xa’, where ‘x’ represents the type of data indicated by ‘q’, ‘v’ and ‘o’, which are short for volume, speed and occupancy, respectively, and ‘a’ represents the detector number. Thus, q1, v1 and o1 denote the volume, average speed and time occupancy in 5 min collected by detector D1, respectively.

Then, the variables coded as ‘Dx ’ represent the difference in ‘x’

between detectors a and b

ab

‘Ax ’ represents the average

, while	ab

value of ‘x’ from detectors a and b.

Fig. 1 Arrangement of detectors

Table 2 Candidate variables considered for modelling Description	Variables

original detector data	q1, v1, o1, q2, v2, o2 q3, v3, o3, q4, v4, o4

As the predictive modelling is treated as a classiﬁcation problem to distinguish crash-prone trafﬁc conditions from normal conditions, the trafﬁc data under conditions that do not lead to a crash are also collected. Similar to the crash data, the non-crash data are collected under different trafﬁc conditions, according to a matched case-control design [3]. For each crash (case) in the dataset, ten corresponding non-crash cases (controls) are randomly determined for the same segment and time in the same month, where no crashes occurred within one hour of the original crash

difference of the traffic flow data between the upstream and downstream detectors

q	v	o	q	v	o

D , D , D , D , D , D

13      13      13      14      14      14

D , D , D , D , D , D

q	v	o	q	v	o

23      23      23      24      24      24

time. Then, trafﬁc data for non-crash cases are extracted from the detectors dataset, and 5794 non-crash data are generated for a crash against non-crash ratio of approximately 1:10.

average value of the traffic flow

data of the upstream and downstream detectors

q	v	o	q	v	o

A , A , A , A , A , A

12      12      12      34      34      34

# Methodology

To address the complexity and over-ﬁtting of the model with

from each detector. Also, the differences and average values of the trafﬁc ﬂow data upstream and downstream of the crash site are calculated from this raw data.

Therefore, the input variables of the model can be divided into three parts: the ﬁrst part is the original detector data; the second part is the difference of the trafﬁc ﬂow data between one upstream detector and one downstream detector; and the third part is the average value of the trafﬁc ﬂow data of two upstream or

excessive variables, the key variables should be selected as input variables before the establishment of the crash prediction model. Besides, the occurrence of crashes is signiﬁcantly related to the trafﬁc states near the crash location. Different combinations of trafﬁc states upstream and downstream of the crash location lead to a different probability of crashes. Thus, the trafﬁc state is classiﬁed into several conditions (free ﬂow, congestion and trafﬁc jam) with a clustering algorithm; and crashes are predicted from

Fig. 2   Structure of the crash prediction model

the input of the different combinations of trafﬁc states. In addition, when predicting the crashes, the SVM model is improved with two penalty parameters to handle unbalanced data. Then, a hybrid crash prediction model combining the SVM model and clustering algorithm, could be obtained with a lower computational load and higher robustness. The entire data ﬂow process is indicated in Fig. 2 and will be described in detail below.

  - Random forest

As shown in Table 2, a total of 30 optional variables can be used to build the model. To avoid a complicated and over-ﬁtted crash prediction model, critical variables should be identiﬁed before model construction. The RF model, which is an ensemble learning method proposed by Breiman, is selected for calculating the importance of variables in the model [21]. In contrast to other traditional variable selection methods, such as logistic regression and CART, the RF model can address the problem of multi-collinearity and high-dimensional (many variables) data due to random sampling. Moreover, the RF model does not require cross-validation because of the formation of out-of-bag (OOB) observations (i.e. the samples that are not extracted from the dataset in the random sampling process and can be used to estimate the generalisation error) [12].

A brief description of the RF model is as follows: For b = 1, …, B,

sample n observations with replacement from the original learning sample L as the sample Lb to build the CART Tb. At each node of the tree, m of the p independent variables is randomly selected to split. Then among the two algorithms implemented in the random forest

R package [25] for calculating variable importance measures, the OOB observations are used to calculate the variable importance

model is a type of learning algorithm based on statistical learning theory and the structural risk minimisation principle, which can be adjusted to map the input–output relationship for a non-linear system. The basic idea of the SVM model is that it maps the input vectors into some high-dimensional feature space Z through non-linear mapping chosen a priori. In this space, a linear decision surface is constructed with special properties that ensure the high generalisation ability of the network [28].

There are many different types of SVM models according to the different objective. Owing to the classiﬁcation problem in this study, the C-support vector classiﬁcation model (C-SVM) in LIBSVM is used in this study [29]. When using the C-SVM model, the data should be divided into a training dataset and a testing   dataset.   The   training   dataset   is   deﬁned   as   vectors

i  i=1

{xi, y }l   , where xi represents the trafﬁc condition variables and yi

= 1 represents the crash occurrence. Therefore, the aim is to ﬁnd

an optimal hyperplane that can separate the category of each input and is deﬁned as

w · x + b = 0	(1)

where w is a vector that is perpendicular to the hyperplane and b is a constant term. Since the purpose of the optimal hyperplane is to maximise the margin between the boundaries of two categories, it can be solved by seeking the maximum margin. However, for the unbalanced data (i.e. the proportion of crashes to non-crashes is close to 1:10) used in this study, several penalty parameters are proposed, and they ﬁnally become a quadratic programming problem including the primal problem

i

i

min 1 wTw + C         j + C        j

and applied in this study as follows [26]: For the CART Tb:

w,b,j 2

+ yi =1

− yi =0

(2) Predict class membership for Lb,OOB using Tb and sum the

i

i

i

(2)

(1) Identify the OOB observations, Lb,OOB = L − Lb.

subject to:    y (wTf(x ) + b) ≥ 1 − j ,

number of times the tree predicts the correct class.

- For independent variables j = 1,…, p:

  - Permute the values of the independent variable xj in Lb,OOB.

  - Use Tb to predict the class membership for Lb,OOB using the permuted xj values; sum the number of times the tree predicts the correct class for the Lb,OOB observations.

ji ≥ 0,	i = 1, .. . , l.

Its dual problem is

−

min 1 aTQa

a 2

eTa

(3)

(iii) Subtract the number of votes for the correct class in the permuted OOB data from the number of votes for the correct class in the OOB data.

The average difference in the accuracy of the OOB observations

against the permuted OOB observations over the B trees is the variable importance measure for xj. Owing to the random sampling process of the RF model for building multiple CARTs to reduce the OOB data, the number of trees in the RF model should be set to 1000 [15].

  - Clustering method

In the ﬁrst part of the hybrid model, a commonly used clustering method, K-means clustering, is adopted to abstract each selected variable into several groups because of the continuous nature of all the variables in this study [27]. A basic description of K-means

yTa = 0

−

subject to: 0 ≤ ai ≤ C+,	yi = 1, 0 ≤ ai ≤ C ,	yi = 0.

where e = [1,…,1]T is the vector of all ones, C+ represents the penalty parameter of the classiﬁcation of crashes, whereas C− is one of the non-crashes, Q is an l × l positive semi-deﬁnite matrix and Qij ≡ yi yj K(xi, xj) and K(xi, xj)= f(xi)Tf(xj) is the kernel function. Here, training vectors xi are mapped into a higher-dimensional space by the function f. Though several kernels have been proposed by researchers, the radial basis function is the most commonly used kernel and is adopted in this study as

i

j

i

j

K(x , x ) = exp(−g1x − x 12),	g . 0	(4)

clustering is summarised as follows.

When given a set of n observations {x1, x2,…, xn}, where each

observation is a d-dimensional vector, the purpose of k-means clustering is to separate the n observations into k sets (k ≤ n) S =

Finally, the decision function that can separate two conditions is

{S1, S2,   …,   Sk}   with   the   within-cluster   sum   of   squares

l

2

i=1

xj Si

j

i

i

argmin    k	1x − c 1 to be minimised, where c is the

sgn

yiaiK(xi, x) + b

(5)

mean of the points in Si.

  - SVM model with two penalty parameters

In the second part of the hybrid model, an SVM model with two penalty parameters is developed for the classiﬁcation of crash-prone conditions from normal trafﬁc conditions. The SVM

i=1

Then, the C-SVM model has three parameters (C+, C−, g) that need to be determined. There is always a globally optimal solution to w and b with the input of parameters (C+, C−, g). The optimal values of parameters C+, C− and g are identiﬁed using a grid search method during the learning process to maximise the model performance [24].

Table 3   Confusion matrix

|  | Predicted crashes | Predicted non-crashes |
| --- | --- | --- |
| real crashes | Tcrash | Fnon_crash |
| real non-crashes | Fcrash | Tnon_crash |

  - Model evaluation

The ratio of the crash data and non-crash data is approximately 1:10 and this becomes an imbalanced classiﬁcation problem. Owing to the small ratio of crash data in the dataset, the overall accuracy metric at

(6) is no longer sufﬁcient to evaluate the performance of a classiﬁcation model because the overall classiﬁcation accuracy can be as high as 90.8%, even if the model classiﬁes all the cases as non-crashes. Hence, several metrics based on the confusion matrix are used for the effectiveness evaluation on the imbalanced classiﬁcation as shown in Table 3.

The sensitivity at (7) and speciﬁcity at (8) are usually used to separately investigate the classiﬁcation performance on two classes, while the sensitivity is the crash classiﬁcation accuracy, and speciﬁcity is the non-crash classiﬁcation accuracy which is a complement of the false alarm rate. In addition, the precision at

(9) and the recall at (10) are often adopted to evaluate the effective detection ability for only one class. The optimally balanced classiﬁcation ability can be observed by G-means at (11), which is the geometric mean of the sensitivity and speciﬁcity, while the F-measure at (12) represents the ability of the model to detect crashes [30]

To select the appropriate number of important variables to develop the crash prediction model, a primary test of the RF model is conducted to obtain the minimum OOB error with fewer variables. In addition, the analysis result shows that with the top eight variables adopted in the RF model, the OOB error reaches a relative minimum. However, it is not always the case that more variables lead to higher prediction accuracy. Then, a comparative analysis is conducted with the construction of the SVM model to examine the possibility of fewer variables based on 408 crashes and 4063 corresponding non-crashes on the Yan-an expressway. Different numbers of input variables for the SVM models are developed for conﬁrming the proper number of input variables. The result shows that models with the four most important variables obtained the best performance and were ultimately selected as the input for the models in this study. As shown in

23	12	3

Figs. 3, the four selected variables are o2, Do , Ao and v , respectively, which is the occupancy of the D2 detector, the occupancy difference of the D2 and D3 detectors, the mean of the occupancy upstream of the crash location, and the average speed of the D3 detector.

In addition, a comparison between the SVM model with and without variable selection is conducted. As mentioned, a training dataset and testing dataset are assigned randomly and equally from a dataset. Generally, the comparison and concerning results are calculated from the testing dataset. However, since over-ﬁtting may be caused by excessive variables, an additional test is conducted with the training dataset to compare the testing dataset results as shown in Table 4. From the table, we know that the SVM model without variable selection exhibits severe over-ﬁtting as the crash prediction accuracy decreases greatly, though the model obtain quite well prediction accuracy with the training

=

Overall accuracy	 	Tcrash + Tnon crash

Tcrash + Fcrash + Fnon crash   + Tnon crash

(6)

dataset. With regard to the SVM model with variable selection, the crash prediction accuracy and the false alarm rate both varies

somewhat, whereas the crash prediction ability which indicated by

Sensitivity = Tcrash/ Tcrash + Fnon crash		(7) Specificity = Tnon crash/ Tnon crash + Fcrash		(8) Precision = Tcrash/ Tcrash + Fcrash	(9)

Recall = Tcrash/ Tcrash + Fnon crash	(10)

F-measure see a minor change. Therefore, it demonstrates that over-ﬁtting can be mitigated greatly with variable.

For the testing results of SVM models with and without variable selection as shown in Table 4, the crash prediction accuracy increases to some degree albeit the false alarm rate increases at the same time. However, the G-means which indicates the balanced

G-mean = s e n s i ti v it y ∗ s p e c i fi c i t y

=

F-measure	2 ∗ precision ∗ recall

precision + recall

(11)

(12)

Therefore, we evaluate the algorithms in terms of the prediction effectiveness using the crash classiﬁcation accuracy, false alarm rate, overall classiﬁcation accuracy, G-means and F-measure. As ﬁnding crash-prone conditions are what we are particularly interested in, the F-measure of the crash prediction model will be used more.

A training dataset and testing dataset are assigned randomly from a dataset. Both datasets contained multiple crashes and non-crashes with their corresponding trafﬁc condition data. To reduce the error caused by the random division of the dataset, ten experiments with different training and testing datasets randomly generated from one set of trafﬁc data are conducted. Then, a set of mean of above-mentioned metrics is used to represent the mode performance.

# Results of analysis

  - Results of variable selection

All 30 variables given in Table 2 are used as the input variables of the RF model to calculate the importance of each variable as shown in Fig. 3, where the horizontal axis represents the average decrease of accuracy caused by the elimination of the variable and which hereby indicates its importance.

Fig. 3 Variable importance: average decrease in accuracy

| Table 4 Results of the different datasets | for different models |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Model |  | Overall accuracy | Crash accuracy | False alarm rate | G-means | F-measure |
| SVM model without variable selection | training dataset | 0.873 | 0.981 | 0.138 | 0.919 | 0.585 |
|  | testing dataset | 0.799 | 0.735 | 0.195 | 0.767 | 0.407 |
| SVM model with variable selection | training dataset | 0.777 | 0.872 | 0.232 | 0.818 | 0.417 |
|  | testing dataset | 0.792 | 0.779 | 0.207 | 0.782 | 0.413 |

Table 5   Results of the hybrid model

|  | Overall accuracy | Crash accuracy | False alarm rate | G-means | F-measure |
| --- | --- | --- | --- | --- | --- |
| hybrid | 0.800 | 0.780 | 0.198 | 0.791 | 0.416 |
| model baseline | 0.792 | 0.779 | 0.207 | 0.782 | 0.413 |

classiﬁcation ability and the F-measure both have a promotion. We thus conclude that variable selection provides more efﬁcient crash prediction.

  - Results of the hybrid model

Based on the data collected on the Yan-an expressway, a comparison analysis of hybrid model used in this study is conducted with the result of the SVM model with variable selection as the baseline. The analysis result is shown in Table 5 as follows.

As we can see from Table 5, the hybrid model shows improved classiﬁcation accuracy compared with the original SVM model. All ﬁve metrics of the hybrid model with k-means clustering have an advantage over the baseline for not only the balanced classiﬁcation ability, but also the ability to predict crashes, which are shown by the G-means and F-measure. Meanwhile, although it is a tiny advancement with the clustering introduced in the hybrid model, the computational property of the SVM model can improve to a large extent, especially with large-scale data.

As represented in the literature review, the prediction accuracy of the hybrid model in this study is superior to that in previous studies, where the model that can predict 73.9% crashes with a false alarm rate of 28.7% in [8] is the best model. In addition, the false alarm rate of the hybrid model performs better than the SVM model in our previous study at 26.1% [19]. Thus, the results show that the hybrid model combined with the SVM model and clustering algorithm is relatively better for the prediction of crashes.

  - Transferability of the model

The transferability of the hybrid model is also conducted in this study to examine whether the prediction model developed on one expressway can be applicable to other similar expressway segments. The data collected on the Shanghai North–South expressway are used to assess the transferability of the models built with the data on the Yan-an expressway. Here, the combined 169 crash data and 1731 non-crash data are chosen as the testing dataset. Meanwhile, the transferability of the model without variable selection and the original SVM model are both tested for comparison. The results of these models are shown in Fig. 4.

As shown in Fig. 4, the three models can predict nearly 60% of the crashes on the other expressway, albeit the accuracy decreases to some extent. Within the three models, the hybrid model with variable selection performs better than the others for all the performance indicators, while the SVM model without variable selection has the worst accuracy. With the use of variable selection and clustering, the crash prediction accuracy increases to some extent, whereas the performance of the false alarm rate decreases a little. It can also be observed from the variety of the G-means and F-measure. It is shown in the ﬁgure that the G-means vary from

0.57 to 0.66, while the F-measure increases by 0.05. Although the transferability of the crash prediction model cannot reach the original prediction accuracy, the transferability in this study already reaches a reasonable extent while a 55.84% crash prediction is obtained using the logistic regression model for the transferability test in the study [6].

# Conclusions and future works

This study aims to improve the performance of real-time crash prediction models using the hybrid model and critical variables selection. First, to promote the crash prediction accuracy as well as the transferability of the crash prediction model with relatively low computational complexity, a hybrid model integrating clustering algorithm and classiﬁcation model is proposed. Second, by selecting critical variables that signiﬁcantly affect the crashes

Fig. 4   Transferability comparisons of the different models

prediction with the RF model, the relationship of crash and trafﬁc states can be understood. A dataset of 577 crashes and 5794 non-crashes combined with the corresponding trafﬁc data 5–10 min before the crashes (non-crashes) are collected on two expressways in Shanghai, while the hybrid model is cross-validated with data collected on the Yan-an expressway, and the transferability of the model is examined with data collected on the North–South expressway. The detailed results are as follows:

- The hybrid model categorises various trafﬁc states with different trafﬁc parameters using k-means clustering and then predicts crashes with an SVM model with two penalty parameters to handle unbalanced crash and non-crash data.

- The results indicate that both the balanced classiﬁcation ability and the crash prediction ability of the hybrid crash prediction model get promoted with a classiﬁcation accuracy of crashes of 78.0% at best. The false alarm rate of the hybrid model also decreases to some extent.

- The RF model is developed to select the four most important variables for the establishment of the crash prediction model. The results show that the ability of the model to predict crashes is enhanced to some degree, with lower over-ﬁtting.

- From the results of the transferability of the three different models, it is found that variable selection and clustering are both advantageous for crash prediction.

Considering the applicability of the real-time crash prediction model, there are another several concerns that deserved further study. Initially, since the weather is also related to crash events, the inﬂuence of adverse weather conditions on crash occurrences is needed to investigate, whereas there are some speciﬁc studies (e.g. [31]) incorporating real-time weather. However, due to the lack of detailed weather data, it is not considered in this paper. On the other hand, how to predict crashes with limited data is another direction for this ﬁeld. For example, regarding roads that do not have dual-loop detectors, only with ﬂow and occupancy data collected from single-loop detectors or trafﬁc speed data collected with other approaches such as sampled ﬂoating cars, the model needs more improvement.

# Acknowledgments

The authors thank the Natural Science Foundation of China (51278362, 51422812), the New Century Excellent Talents in University (NCET13-0425) and the Fundamental Research Funds for the Central Universities for supporting this research. The authors also thank the anonymous reviewers and the editor for their valuable comments.

# References

- Oh, C., Oh, J.S., Ritchie, S.G., et al.: ‘Real-time estimation of freeway accident likelihood’. Presented at the 80th Annual Meeting of the Transportation Research Board, Washington, DC, 2001

- Lee, C., Hellinga, B., Saccomanno, F.: ‘Real-time crash prediction model for application to crash prevention in freeway trafﬁc’, Transp. Res. Rec., 2003, 1840, pp. 67–77

- Abdel-Aty, M., Uddin, N., Pande, A., et al.: ‘Predicting freeway crashes from loop detector data by matched case-control logistic regression’, Transp. Res. Rec., 2004, 1897, pp. 88–95

- Abdel-Aty, M., Pande, A., Lee, C., et al.: ‘Crash risk assessment using intelligent transportation systems data and real-time intervention strategies to improve safety on freeways’, J. Intell. Transp. Syst., 2007, 11, pp. 107–120

- Abdel-Aty, M., Uddin, N., Pande, A.: ‘Split models for predicting multivehicle crashes during high-speed and low-speed operating conditions on freeways’, Transp. Res. Rec., 2005, 1908, pp. 51–58

- Pande, A., Nuworsoo, C., Shew, C.: ‘Proactive assessment of accident risk to improve safety on a system of freeways’. No. MTI Report 11–15, 2012

- Abdel-Aty, M., Hassan, H.A., Ahmed, M., et al.: ‘Real-time prediction of visibility related crashes’, Transp. Res. C, Emerg. Technol., 2012, 24, pp. 288–298

- Abdel-Aty, M., Pande, A.: ‘Identifying crash propensity using speciﬁc trafﬁc speed conditions’, J. Safety Res., 2005, 36, (1), pp. 97–108

- Pande, A., Abdel-Aty, M.: ‘Assessment of freeway trafﬁc parameters leading to lane-change related collisions’, Accident Anal. Prev., 2006, 38, (5), pp. 936–948

- Golob, T., Recker, W., Pavlis, Y.: ‘Probabilistic models of freeway safety performance using trafﬁc ﬂow data as predictors’, Safety Sci., 2008, 46, (9), pp. 1306–1333

- Zheng, Z., Ahn, S., Monsere, C.M.: ‘Impact of trafﬁc oscillations on freeway crash occurrences’, Accident Analysis and Prevention, 2010, 42, (2), pp. 626–636

- Hossain, M., Muromachi, Y.: ‘A Bayesian network based framework for real-time crash prediction on the basic freeway segments of urban expressways’, Accident Anal. Prev., 2012, 45, pp. 373–381

- Hossain, M., Muromachi, Y.: ‘A real-time crash prediction model for the ramp vicinities of urban expressways’, IATSS Research, 2013, 37, (1), pp. 68–79

- Xu, C., Tarko, A.P., Wang, W., et al.: ‘Predicting crash likelihood and severity on freeways with real-time loop detector data’, Accident Anal. Prev., 2013, 57, pp. 30–39

- Xu, C., Wang, W., Liu, P.: ‘A genetic programming model for real-time crash prediction on freeways’, IEEE Trans. Intell. Transp. Syst., 2013, 14, (2), pp. 574–586

- Xu, C., Wang, W., Liu, P., et al.: ‘Using the Bayesian updating approach to improve the spatial and temporal transferability of real-time crash risk prediction models’, Transp. Res. C, Emerg. Technol., 2014, 38, pp. 167–176

- Xu, C., Wang, W., Liu, P., et al.: ‘Development of a real-time crash risk prediction model incorporating the various crash mechanisms across different trafﬁc states’, Trafﬁc Inj. Prev., 2014, 16, (1), pp. 28–35

- Yu, R., Abdel-Aty, M.: ‘Utilizing support vector machine in real-time crash risk evaluation’, Accident Anal. Prev., 2013, 51, pp. 252–259

- Sun, J., Sun, J., Chen, P.: ‘Use of support vector machine models for real-time prediction of crash risk on urban expressways’, Transp. Res. Rec., 2014, 2432, pp. 91–98

- Yeo, H., Jang, K., Skabardonis, A., et al.: ‘Impact of trafﬁc states on freeway crash involvement rates’, Accident Anal. Prev., 2013, 50, pp. 713–723

- Breiman, L.: ‘Random forests’, Mach. Learn., 2001, 45, (1), pp. 5–32

- Geng, G., Li, N., Gong, S.: ‘Feature selection method for network intrusion based on fast attribute reduction of rough set’. Int. Conf. on Industrial Control and Electronics Engineering, 2012, Xian, China, pp. 530–534

- Li, X., Lord, D., Zhang, Y., et al.: ‘Predicting motor vehicle crashes using support vector machine models’, Accident Anal. Prev., 2008, 40, (4), pp. 1611–1618

- Li, Z., Liu, P., Wang, W., et al.: ‘Using support vector machine models for crash injury severity analysis’, Accident Anal. Prev., 2012, 45, pp. 478–486

- Liaw, A., Wiener, M.: ‘Classiﬁcation and regression by random forest’, R News, 2002, 2, (3), pp. 18–22

- Archer, K.J., Kimes, R.V.: ‘Empirical characterization of random forest variable importance measures’, Comput. Stat. Data Anal., 2008, 52, (4), pp. 2249–2260

- Hartigan, J.A., Wong, M.A.: ‘Algorithm AS 136: a k-means clustering algorithm’,

J. R. Stat. Soc. Series C (Appl. Stat.), 1979, 28, (1), pp. 100–108

- Cortes, C., Vapnik, V.: ‘Support-vector networks’, Mach. Learn., 1995, 20, (3), pp. 273–297

- Chang, C.C., Lin, C.J.: ‘LIBSVM: a library for support vector machines’, ACM Trans. Intell. Syst. Technol., 2011, 2, (3), p. 27

- Tang, Y., Zhang, Y.Q., Chawla, N.V., et al.: ‘SVMs modeling for highly imbalanced classiﬁcation’, IEEE Trans. Syst. Man Cybern. B, Cybern., 2009, 39, (1), pp. 281–288

- Yu, R., Abdel-Aty, M., Ahmed, M.: ‘Bayesian random effect models incorporating real-time weather and trafﬁc data to investigate mountainous freeway hazardous factors’, Accident Anal. Prev., 2013, 50, pp. 371–376